|  | Date: 10/09/2019 |  |  |  |
| --- | --- | --- | --- | --- |
| Project: | McMahon Carwash |  |  |  |
| # Corrective Action Log |  |  |  |  |
| Number | Corrective Action Needed | Date Action Noted | Date Corrected | Date Correction Noted By Inspector |
| 1 | Post the NOI | 02/14/19 |  | 02/25/19 |
| 2 | Post a rain gauge | 02/14/19 |  |  |
| 3 | Ensure there is a spill kit | 02/14/19 |  |  |
| 4 | Install a rock pad entrance to minimize the trackout of sediment onto the streets. | 02/14/19 |  | 03/12/19 |
| 5 | Provide SWPPP book on site | 02/14/19 |  | 02/25/19 |
| 6 | The silt fence at the SE corner of the site needs to be reinstalled. It is laying on the ground where it should be installed. | 02/25/19 |  | 03/26/19 |
| 7 | Need to make sure the silt fence around the site is toed in ( bottom of fence is buried 6") | 2\2519 |  | 07/16/19 |
| 8 | Need to read one training cession per month from tab 7 and sign it and date it. | 2/25/19 |  |  |
| 9 | Reinstall 5 panels of the security fence on the west side have fallen by strong winds. | 03/12/19 |  | 03/26/19 |
| 10 | Around the concrete batch plant the concrete powder needs to be removed from the ground and disposed of. | 03/26/19 |  | 04/10/19 |
| 11 | No new issues at the time of this inspection | 04/10/19 |  | 04/10/19 |
| 12 | Re install approx. 50' of silt fence at the west side of the site behind the batch plant. | 05/07/19 |  | 07/16/19 |
| 13 | No new issues at the time of this inspection. Address previous issues | 05/21/19 |  | 05/21/19 |
| 14 | Reatach the silt fence to the chain link fence where needed on the North side of the site | 06/04/19 |  | 07/16/19 |
| 15 | The trash bin needs to be emptied | 06/04/19 |  | 07/02/19 |
| 16 | Replace the concrete washout. Washout pits must have a liner. 07/16/19 It has been cleaned. | 06/04/19 |  | 07/16/19 |
| 17 | No new issues at the time of this inspection. Address previous issues | 06/18/19 |  | 06/18/19 |
| 18 | No new issues at the time of this inspection. Address previous issues | 07/02/19 |  | 07/02/19 |
| 19 | Repair and reinstall the silt fence along the site perimeter | 07/16/19 |  |  |
| 20 | Re-locate the rock pad entrance.It is not being used and they are jumping on one side of the curb. | 07/31/19 |  |  |
| 21 | Remove the blow away trash at the East side of the site . (Stucco guys trash ) | 07/31/19 |  | 08/14/19 |
| 22 | No new issues at the time of this inspection. Address previous issues | 08/14/19 |  | 08/14/19 |
| 23 | The trash bin needs to be emptied | 08/28/19 |  | 09/11/19 |
| 24 | No new issues at the time of this inspection. Address previous issues | 09/11/19 |  | 09/11/19 |
| 25 | Cleanup the concrete washed out at the East side of the site | 09/25/19 |  |  |
| 26 | Remove the blow away trash around the building. | 09/25/19 |  |  |
| 27 | No new issues at the time of this inspection. Address previous issues | 10/09/19 |  |  |
| SWPPP Management Signature: Date: |  |  |  |  |