September 12, 2013

Mike Walla, P.E.

Walla Engineering

6501 Americas Parkway NE Suite 301

Albuquerque, NM 87110

Re:	Drive-Through land addition at Corrales Shopping Center,

Grading and Drainage Plan

Engineer’s Stamp Date 8-21-13 (A14D001A)

Dear Mr. Walla,

Based upon the information provided in your submittal received 8-20-13, the above referenced plan cannot be approved for Building Permit until the following comments are addressed:

- The City cannot approve grading in the NMDOT ROW without NMDOT approval.  Hydrology recommends: for Pond II- move the 19.5 contour out of the NMDOT ROW and for Pond I-lowering the trash enclosure and only grading the 16 and 16.5 contour.  The 16.5 contour should be proposed at or inside the NMDOT ROW.

- It is not clear how parking lot flows enter Pond I.  A curb cut should be provided in the parking lot near Pond I to allow stormwater to enter Pond I.

- Per a site visit 9-12-13, the North Pond between the dirt path and the pavement is relatively flat.  It would be beneficial if you could grade a path from the end of the concrete rundown to the invert of the PVC pipes and provide a distance so that the contractor could place the PCV culverts in the low point of the ponding area.

- Show the wall and accompanying space in the bottom course of the wall from the Administrative Amendment.

If you have any questions, you can contact me at 924-3986.

Sincerely,

Curtis Cherne, P.E.

Principal Engineer, Planning Dept.

Development Review Services

C:	e-mail