This is the layout of the WSPGW input data file, (Study_Name.wsw).  The file must be

in ASCII Text format with no hidden characters other than a line feed at the end of each

line, and have no blank lines.  The actual input lines in this sample file are in bold

font format.  This data file is normally prepared by the WSPGW program input data

editor, however, you may use a word processor such as Windows Notepad or Wordpad

to edit and/or construct the file.  The file must begin with the T1, T2 and T3 title lines,

then with the SO System Outlet Line.

First 2 Columns Signify Type of Element Line

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

T1 Title Line Data - Line Number 1                                             0

Column 80  0=English Units, CFS and Feet Dimensions							    	    1=Metric Units, CMS, Meters

2=English Units, GPM, Pipes in Inches

4=Metrics Units, Liters/S, Pipes in CM

T2 Line 2 The Lines May Be Up To 80 Characters Long Except T1 Line

T3 Line 3

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

SO   1000.0008800.000  1                          8815.000

System Outlet		Columns 5 - 13 Station with Decimal Point in Column 10

Columns 14 - 21 Elevation with Decimal Point in Column 18

Columns 22 - 24 Channel Definition Number

Columns 51 - 58 Known Water Surface Elevation at System Outlet,

Normally, set this the same as invert elevation unless a known

water depth is available.

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

R    1200.0008815.000  1      .030                                25.000

Reach (Irregular Channel)	Columns 5 - 13 Station with Decimal Point in Column 10

Columns 14 - 21 Elevation with Decimal Point in Column 18

Columns 22 - 24 Channel Definition Number

Columns 31 - 34 Manning's "N" with Decimal Point in Column 31

Columns 65 - 72 Angle of Curve in reach + or -

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

TS   1300.0008820.000  2      .015                                11.000

Transition Structure	Columns 5 - 13 Station with Decimal Point in Column 10

Columns 14 - 21 Elevation with Decimal Point in Column 18

Columns 22 - 24 Channel Definition Number

Columns 31 - 34 Manning's "N" with Decimal Point in Column 31

Columns 65 - 72 Angle of Curve in transition + or -

(+ = Right or - = Left looking Upstream)

Columns 74 - 80 Angle Point at Upstream End  + or -

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

BX   1300.0008820.000  3

Bridge Exit			Columns 5 - 13 Station with Decimal Point in Column 10

Columns 14 - 21 Elevation with Decimal Point in Column 18

Columns 22 - 24 Channel Definition Number

(Must be same size as previous CD and, have pier(s))

Note:  A Wall Exit line (WX) has the same format as a BX, except the previous Channel Definition

downstream must be of a larger cross section than the WX cross section.

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

R    1350.0008822.000  3      .015                               -15.000    .000 0

Reach (See Above)

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

JX   1360.0008822.400  3  4  5.015  500.000  600.0008824.0008825.000-45.0 60.0  -2.865

Junction			Columns 5 - 13 Station with Decimal Point in Column 10

Columns 14 - 21 Elevation with Decimal Point in Column 18

Columns 22 - 24 Channel Definition Number (U/S end of JX)

Columns 25 - 27 Channel Definition Number (Lateral #1)

Columns 28 - 30 Channel Definition Number (Lateral #2)

Columns 35 - 43 Flow Rate from Lateral #1, Decimal Pt. Column 40

Columns 44 - 52 Flow Rate from Lateral #2, Decimal Pt. Column 49

Columns 53 - 60 Invert Elevation Lateral #1, Decimal Pt. Column 57

Columns 61 - 68 Invert Elevation Lateral #2, Decimal Pt. Column 65

Columns 69 - 73 Confluence Angle Lateral #1, Decimal Pt. Column 72

Columns 74 - 78 Confluence Angle Lateral #2, Decimal Pt. Column 77

Columns 81 - 86 Angle of Curve in JX, Decimal Pt. Column 83

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

R    1400.0008880.000  3      .015                                45.000    .000 0

Reach (See Above)

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

BE   1400.0008880.000  2       .20

Bridge Entrance		Columns 5 - 13 Station with Decimal Point in Column 10

Columns 14 - 21 Elevation with Decimal Point in Column 18

Columns 22 - 24 Channel Definition Number

(Must be same size as previous CD and, not have pier(s))

Columns 31 - 34 Minor Loss Pier Factor, Decimal Point Column 32

Note:  A Wall Entrance line (WE) has the same format as a BE, except the previous Channel 	Definition downstream must be of a smaller cross section than the BE cross section.

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

TS   1450.0009000.000  6      .014                               -20.000

Transition (See Above)

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

SH   1450.0009000.000  6                          9000.000

System Headworks		Columns 5 - 13 Station with Decimal Point in Column 10

Columns 14 - 21 Elevation with Decimal Point in Column 18

Columns 22 - 24 Channel Definition Number

(Must be same size as previous CD )

Columns 51 - 58 Known Water Surface Elevation, Decimal Point Column 55

Normally, set this the same as invert elevation to start

calculation at critical depth, unless a known water depth is 					available, such as normal depth.  Note:  The program will accept

a very low depth here shuch as a sluice gate water surface.

Note:  Following the System Headworks (SH)line, Channel Definition (CD), Cross Section (PTS),

and flow rate (Q) lines are required to complete the file.

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

CD   2  1   0    .000  30.000   50.000 2.000 2.000   .00

Channel Definition		Columns 4 - 6 Channel Definition Serial Number (up to 200)

Column  9  Type of Channel

1 - Trapezoidal

2 - Rectangular

3 - Box or Covered Trapezoidal

4 - Pipe(s)

5 - Irregular Open (Requires PTS lines)

6 - Irregular Covered (Requires PTS Lines)

Columns 12 - 13 Number of Piers or Pipes

Columns 14 - 21 Average Width of Piers - Channel Types 1,2,3,5,6

Columns 22 - 29 Height of Channel or Diameter of Pipes

Columns 30 - 38 Width of Channel

Columns 39 - 44 Left Side Slope of Channel Types 1 or 3

Columns 45 - 50 Right Side Slope of Channel Types 1 or 3

Columns 51 - 56 Invert Cross Fall(-) of Channel Types 1, 2 or 3

CD   3  3   2   2.000  30.000   50.000 2.000 2.000   .00

Channel Definition (See Above)

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

Channel Definition (See Above)

CD   4  4   2    .000   6.000     .000  .000  .000   .00

CD   5  3   0    .000   4.000    8.000 1.000 1.000-12.00

CD   6  2   0    .000  30.000   50.000  .000  .000-12.00

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

CD   1  5   3   1.000           12.00 15.00 14.00   .00   .00   .00   .00   .00   .00   .00

Channel Definition		Columns 4 - 6 Channel Definition Serial Number (up to 200)

(Irregular)			Column  9  Type of Channel

5 - Irregular Open (Requires PTS lines)

6 - Irregular Covered (Requires PTS Lines)

Columns 12 - 13 Number of Piers or Pipes (Up to 10 piers, 99 pipes)

Columns 14 - 21 Average Width of Piers - Channel Types 1,2,3,5,6

Columns 32 - 37 Height of Pier relative to PTS data

Columns 38 - 43, 44 - 49, 50 - 55, 56 - 61, 62 - 67, 68 - 73,

74 - 79, 80 - 85, 86 - 91  Same Height Data as Above

For up to 10 piers.

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

PTS  1 6   .000 50.000 10.000 30.000 50.000 10.000 60.000 30.000100.000 50.000

PTS     120.000 60.000

Channel Cross Section	Columns 4 - 6 Channel Definition Serial Number (up to 200)

(Irregular)			Columns 7 - 8 Number of Cross Section Points (Up to 35) 1st Line Only

Columns 9 - 15 X - Value

Columns 16 - 22  Y - Value (Max value 999.999)

Columns 23 - 29  X - Value (Max Value 999.999)

Columns 30 - 36  Y - Value, ....

Columns 37 - 43  X - Value,  ....

Note - Up to 5 X-Y Values may be entered per each PTS line in

the columns above and 44 - 50, 51 - 57, 58 - 64, 65 - 71, 72 - 78.

0        1         2         3         4         5         6         7         8         9

123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890

Q          5000.000   .0

Q          7000.000  1.2

Flow Rates			Columns 11 - 19 System Headworks Flow Rate, Decimal Point in Column 16				Columns 20 - 24 Q-Factor - Lateral Flow Rates in Junctions are multiplied

by this value.

Note:  Any number of Flow Rate (Q) lines may be entered.