March 22, 2012

David B. Thompson, P.E.

Thompson Engineering Consultants, Inc.

P.O. Box 65760

Albuquerque, NM 87193

Re:	Amendment to the Trails Drainage Master Plan

Engineer’s Stamp Date 1-25-12 (file no. C09-D001)

Dear Mr. Thompson,

Based upon the information provided in your submittal engineer stamp dated and received 1-25-12, the above referenced Drainage Master Plan and Drainage Report (“DMP”) for the Trails Development (“Trails”) is not approved.  The following comments or issues are outstanding and need to be addressed:

- The City’s Development Process Manual (“DPM”) requires that storm water generated from a developed site retain the 100 year, 10 day storm for an interim condition.  Pond “K” does not appear to have adequate capacity to accommodate the flows generated by the Trails as it exists today.  Please verify that pond “K” does have the capacity for the storm flows identified above or identify how these flows are to be controlled and when.

- The Trails is required to drain ponds “K” and “J” in order to continue to develop.    A storm drain pipe from the two ponds is required to outfall into the Boca Negra Dam.  This report is required to provide a schedule for the construction of this pipe.

- This report proposes to drain the Trails to the east.   The Boca Negra Agreement requires a storm drain in Universe Blvd from the Trails to the Boca Negra Detention Dam.  This report is inconsistent with that Agreement. Accordingly, this proposal will need to be amended consistent with the Boca Negra Agreement as a condition to the implementation of this proposal.

- Provide the volume required, volume provided, and water surface elevation for each pond on the plan and also show existing and proposed grades in the vicinity of the pond.

March 22, 2012

Trails Master Drainage Plan

Page 2

- The City is concerned with the excessive amount of ponds for and within the Trails.  Provide a solution that does not include a Pond F.  The drainage from Paseo del Norte Blvd. and basin F7 should drain to Universe Blvd.  Please provide analysis points for the storm drain in Universe Blvd. where flows are introduced into this system.

- Please investigate discharging the storm water generated by the western 1/4 to 1/3 of the Trails south directly into the Boca Negra Arroyo west of the APS site.

- To reduce the number of interim ponds, the storm drain in Universe Blvd. from basin J9 should drain south into one large pond built to City standards.  This pond should be located at or near the southeast corner of the Trails.  This pond will be permitted to have only one inlet and one outlet storm drain and therefore act as a detention pond that drains to the Boca Negra Dam.

- Regarding the pond referred to in item number 7, please provide a volume calculation for the currently approved drainage plan and for a scenario that diverts the western 1/4 to 1/3 of the Trails development as discussed above in item number 6.

If you have any questions, you can contact me at 924-3986.

Sincerely,

Curtis Cherne, P.E.

Principal Engineer, Planning Dept.

Development and Building Services

Cc:  file