THE TRAILS check list  ( 5/20/2014 ~ Greg Olson’s field notes)

- Universe at Oakridge:  Do the Inlets on Oakridge stand full of water?  As-Builts show NO OUTFALL line – YES (but not when it doesn’t rain).  They have no outfall from the manhole in Universe, and they are currently full of tumbleweeds and trash (5/20/14).

- Universe south of Oakridge:  Does the Upper Pond J have any inlet pipe?  If so, where and how big, and who built it?  (no As-built) – No inlet pipe.  The west side of the pond has been breashed, north of the spillway, for +25’ either side of the PNM H-pole.

- Universe at Treeline:  What size is the inlet to lower Pond J ? (60” per 691282) – 60” spigot end; water marks suggest that the pipe is sloped toward the pond.

- Universe south of Treeline, across from Kayser Mill, is there an inlet line to the Swale at the NW corner of Pond K?  If so, what size and treatment? – NO inlet

- Universe between Kayser Mill and Woodmont:  Is there a 30” inlet to Pond K, and if so, what size and end treatment? – YES 30” inlet line, bell end of pipe is +16’ east of the manhole, with no end section,  bank around pipe is eroding.

- Universe south of Woodmont:  Confirm the 42” inlet line to southern Pond K, what size and what end treatment? – 42” tapered end section with steel grate.  There is a lot of trash behind the grate.

- Universe south of Woodmont:  What are the limits (Sta. 11+52 - Sta. 12+20 ) of the 48” line NOT BUILT in Universe Boulevard? – The manholes at the above stations both have a section of pipe stubbed out +8-feet in the direction of the missing segment, with CMU a wall at the end of the stub.  Note that the northern MH (Sta. 12+20) also has a 42” or 48” pipe stubbed east, with a CMU plug, not shown on any plans.)

- PHOTOS:

  - Pond E:  Photo inside to see existing 12” (rectangular) Orifice Plate

  - Pond H:  Photo inside = No Plug

  - Pond G:  Photo inside SE MH = No Plug

  - Pond J Inlets   ( 1, 2 and 3 above )

  - Pond K inlets   ( 4, 5 and 6 above )