October 14, 2011

James D. Hughes, P.E.

Mark Goodwin & Associates, PA

P.O. Box 90606

Albuquerque, NM 87199

Re:  The Team Office Building Grading and Drainage Plan

Conceptual (C-12/D026)

Dear Mr. Hughes,

Based upon the information provided in your submittal dated 10-14-2011, the above referenced conceptual grading and drainage plan should address the following concerns:

- It appears the outfall on the southern edge of the property will require approval from

the State Highway Department before it can be utilized in the drainage plan.

- The right-of-way on the western edge of the property is county owned and any

required roadway improvements will need to be addressed prior to construction.

- The outflow from Basin 102 of 22.8 cfs does not match the figure in the North Coors

DMP of 8 cfs for Basin 13.1W, please clarify this value.

- The DMP for the area requires that sediment ponds be maintained on site to manage

the sediment levels eventually entering the Corrales Canal to the east.

- How does the proposed detention pond adequately mitigate both accepting the offsite

flows from Basins 103 and 104 as well as any increases in onsite runoff? Please

provide pond calculations.

- How will the outfall on the southern edge of the property be protected from offsite

flows?

- Please provide sidewalk culvert details for the central sidewalk portion of the project.

Will it be covered?

- All benchmarks and elevations should be referenced to NAVD 88 standards.

- Please provide a FIRM panel be placed on the drainage plan.

- Please delineate street names on drainage plan.

If you have any questions, you can contact me at 924-3695.

Sincerely,

Shahab Biazar, P.E.

Senior Engineer, Planning Dept.

Development and Building Services

# C:file

CJH