April 30, 2013

Ronald R. Bohannan, P.E.

Tierra West, LLC

5571 Midway Park Pl NE

Albuquerque, NM 87109

Re:	Albuquerque Sporting Center, Tract A-1, Albuquerque West Unit 2

Grading and Drainage Plan

Engineer’s Stamp dated 4-26-13 (C13/D029)

Dear Mr. Bohannan

Based upon the information provided in your submittal received 4-26-13, the above referenced report can not be approved for Site Plan for Building Permit based on the following comments:

- Please provide a copy of the basin map used in Smith Engineering drainage study.

- Based on the report it is determined that the 77.76 cfs is greater than the 100-yr/6-hour storm based on the land treatments Smith Engineering had used in their analysis.  Please provide calculations.

- Previously we had asked for a section at its existing conditions including the water surface elevation just west of the project.  This section is shown at proposed conditions.  No water surface elevation was shown.  Please provide calculations.

- The runoff can not be routed to the street passed Phase-I construction.

- It appears more runoff than 30.81 cfs may enter the 20’ opening fronting Phase-I construction.  How was this determined?  Provide calculations.

- The channel can  not encroach into the cul-de-sac.

The following comments should be addressed prior to Building Permit Approval:

- Approval from NMDOT will be required for the work along Paseo Del Norte.

- Plates 22.3 D-5 and 22.3 D-6 should be used in determination of the inlets flow capacity.

- Was scouring assumed where the runoff enters the channel at the end of the concrete rundown?  The runoff down may have to be extended below the riprap.  Provide calculation for the concrete rundown.

Ronald R. Bohannan, P.E.

Page 2

April 24, 2013

The following comments should be addressed prior to Building Permit Approval:

- Approval from NMDOT will be required for the work along Paseo Del Norte.

- Flows at the proposed pipes appear to be at inlet control.  Please provide calculations to assure adequate head is provided.  Will there be end sections?

If you have any questions, you can contact me at 924-3695.

Sincerely,

Shahab Biazar, P.E.

Senior Engineer, Planning Dept.

Development and Building Services

C:	e-mail