July 14th, 2014

Ron Hensley, P.E.

THE Group

300 Branding Iron Rd SE

Rio Rancho, NM 87124

Re:	Sevano Place Subdivision Drainage report and Grading Plan

Grading and Drainage Plan

Engineer’s Stamp Date 6-20-14 (C18D075)

Dear Mr. Hensley,

Based upon the information provided in your submittal received 6-20-14, the above referenced report and plan cannot be approved for Preliminary Plat action by the DRB until the following comments are addressed:

- The drainage report is to state how the project will manage the first flush.  A depressed front yard would be sufficient.  This could be shown with one spot elevation on the grading plan and a detail for a typical lot.

- The AHYMO documentation in the report is for the 24-hour storm rather than the 6-hour storm.  In addition, the hydrograph for a 6-hour storm has a higher peak flow than the 24-hour storm.  The 6-hour storm is to be used for street capacity and sizing drainage infrastructure.

- The outfall to Signal Ave. should be designed so that there is a maximum of two -2 foot sidewalk culverts.  The street may have to be super-elevated with standard curb to accomplish this.  Hydrology did not see calculations in the report for this outfall.  There should be a gap in the wall for an emergency spillway.

- Hydrology has a similar comment for the outfall to Alameda Blvd.  Provide calculations for the inlet and outfall pipes to the trunk in Alameda.  The street should have a cross-slope, possibly super-elevation to the west.

- Provide the EGL where the water is running west (4.0%) then has to make a 90 degree turn.

- Will driveway locations on Lots 1 and 10 interfere with the super-elevating the stub-streets due to the water depth?

- Show temporary asphalt on the grading plan sheet 2 of 2 in between the existing edge of asphalt and the new curb.

- The inlet on Signal should be a Type A since it is the furthest upstream.

- A retaining wall should be proposed on the north side of Lot 18.

- Is the sidewalk shown in the correct location on Alameda Blvd?  Usually the back of sidewalk is at the property line.

- Revise the table on p. 4 to show basins 117.314 and 117.312 are 90% D 10% C as they are streets

- Hydrology calculated the runoff for basin 117.311 as 13.6 cfs, rather than the 12.7 shown on the tables on p. 7 and 8.

If you have any questions, you can contact me at 924-3986.

Sincerely,

Curtis Cherne, P.E.

Principal Engineer, Hydrology

Planning Dept.

C:	e-mail