| BERNALILLO COUNTY DRAINAGE MASTER PLAN TABLE A - CULVERT CROSSINGS NORTH ALBUQUERQUE ACRES - SANDIA HEIGHTS SOUTH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Road Name | Location Number | Type | Dimensions | Maximum Headwater Depth (ft) | Capacity (cfs) | 100-year Discharge Rate (cfs) | Velocity X Depth(1) | Condition | Comments |
| Zone Atlas Page B-21 |  |  |  |  |  |  |  |  |  |
| Eubank Boulevard | B21-01 | CMP | 24" | 2.6 | 16 | <10 | NA | Light brush in approach |  |
| Zone Atlas Page C-22 |  |  |  |  |  |  |  |  |  |
| Access Road | CV-C22-15 STA 14+40 | RCP | 24" | 2.9 | 22 | 15 | NA | Light sediment at discharge end |  |
| Anaheim Avenue | CV-C22-04 STA 60+15 | RCP | 2 - 42" | 4.7 | 140 | 92 | NA | Light brush and sediment at entrance |  |
|  | CV-C22-07 STA 57+50 | CMP | 24" | 5.5 | 30 | 10 | NA | Light brush at entrance |  |
|  | CV-C22-08 STA 52+25 | CMP | 24" | 3.3 | 20 | 5 | NA | Light brush and sediment in entrance |  |
| Cedar Hill Road | CV-C22-13 STA 0+25 | RCP | 24" | 2.4 | 15 | < 10 | NA | Grassy approach channel |  |
| Corona | CV-C22-16 STA 31+35 | RCP | 24" | 2.5 | 18 | 6 | NA |  |  |
| Lowell Street | CV-C22-01 STA 22+20 | CMP | 3 - 30" | 5.2 | 120 | 68 | NA | Brush obstructing entrance |  |
|  | CV-C22-02 STA 18+85 | CMP | 30" | 3.4 | 28 | 34 | (2) | Brush obstructing entrance | $Avulsion |
|  | CV-C22-05 STA 27+55 | RCP | 5 - 30" | 4.3 | 450 | 150 | NA | Approach clear |  |
|  | CV-C22-06 STA 32+60 | CBC | 4 - 10'  6' | 8.4 | 2200 | 1292 | NA | Approximately 1' sediment in invert |  |
| Oakland Avenue | CV-C22-10 E. of Tennyson | CMP | 24" | 4.1 | 22 | < 10 | NA | Nearly 2 silted in. |  |
| Signal Avenue | CV-C22-17 STA 0+25 | RCP | 30" | 3 | 32 | 7 | NA |  |  |
| Tennyson Avenue | CV-C22-09 STA 115+10 | CMP | 24" | 4.7 | 26 | < 10 | NA | Brush and debris in approach |  |
|  | CV-C22-11 STA 103+30 | CMP | 36" | 3.9 | 45 | 15 | NA | Grassy approach channel |  |
|  | CV-C22-12 STA 95+55 | CMP | 48" | 5.8 | 110 | 372 | Avulsion | Approach channel clear | $Avulsion |
|  | CV-C22-14 STA 88+10 | CBC | 2 - 10'  6' | 9.0 | 1240 | 1207 | NA | Channel clear |  |
| Zone Atlas Page C-23 |  |  |  |  |  |  |  |  |  |
| Bighorn Ridge Drive | CV-C23-04 STA 29+90 | RCP | 30" | 3.2 | 34 | 405 | 5.3 | Entrance nearly half blocked with sediment | $d > 0.67' |
|  | CV-C23-50 STA 25+95 | RCP | 24" | 3.7 | 26 | 8 | NA | Heavy brush at entrance and discharge area |  |
|  | CV-C23-51 STA 17+40 | RCP | 24" | 3.6 | 26 | 17 | NA | Light brush at entrance |  |
| Bighorn Ridge Road | CV-C23-08 STA 11+75 | RCP | 2 - 36" | 4.5 | 120 | 668 | 7.6 | Brush in approach barrels clean | $V  d > 6 $d > 0.67' |
|  | CV-C23-09 STA 15+00 STA 15+70 | RCP | 1 - 24" 1 - 48" | 11 12 | 55 220 275 Total | 98 | NA | Light brush at entrance to 48" pipe |  |
|  | CV-C23-10 STA 19+55 | RCP | 48" | 7.1 | 150 | 105 | NA | Debris and brush at pipe entrance |  |
|  | CV-C23-53 STA 30+55 | RCP | 30" | 3.8 | 38 | < 10 | NA | Approach clear, rock debris at entrance |  |
|  | CV-C23-54 STA 24+40 | RCP | 15" | 5.6 | 15 | < 10 | NA | Heavy sedimentation in approach channel and pipe entrance (nearly plugged) |  |
| Black Bear Loop | CV-C23-31 STA 18+40 | RCP | 18" | 2.8 | 13 | < 10 | NA | Entrance nearly 2 silted in, heavy brush in approach channel and discharge area |  |
|  | CV-C23-34 STA 30+05 | CMP | 24" | 4.0 | 22 | 55 | Avulsion (see maps) | Entrance nearlya silted in, light brush in approach channel | $ HW/D > 1.5 $Avulsion |
|  | CV-C23-39 STA 26+40 | RCP | 18" | 2.7 | 13 | 9 | NA | Entrance a to 2 silted in, heavy brush at each end |  |
|  | CV-C23-52 STA 15+40 | RCP | 12" | 2.5 | 6 | 28 | 1.2 | Entrance nearly plugged with sediment | $ HW/D > 1.5 |
| Black Bear Road | CV-C23-02 STA 28+65 | RCP | 2 - 24" | 3.4 | 52 | 720 | Avulsion (see maps) | Brushy approach channel | $ HW/D > 1.5 $Avulsion |
|  | CV-C23-03 STA 12+40 | CMP RCP | 2 - 39"  30" 2 - 24" | 3.3 3.2 | 67 46 113 Total | 1173 | Avulsion (see maps) | Light brush in approach channel | $ HW/D > 1.5 (RCP) $Avulsion |
|  | CV-C23-07 STA 35+35 STA 36+25 | RCP | 2 - 15" | 1.4 | 17 | 514 | 5.0 | Pipes silted in | $ HW/D > 1.5 $d > 0.67' |
|  | CV-C23-30 STA 30+20 | RCP | 15" | 2.7 | 10 | 20 | 0.23(2) | Nearly completely silted in - heavy brush and sediment at discharge end | $ Avulsion from CV-C23-02 |
|  | CV-C23-35 STA 41+80 | RCP | 18" | 2.9 | 14 | 45 | 1.9 | Entrance silted in, heavy brush in approach channel |  |
| Black Hawk Drive | CV-C23-20 STA 27+60 | CMP | 2 - 24" | 2.8 | 34 | 44 | 0.06 | North culvert entrance damaged, both pipe entrances 2 blocked with sediment |  |
|  | CV-C23-47 STA 31+45 | RCP | 24" | 3.6 | 26 | 25 | NA | Pipe silted in |  |
|  | CV-C23-48 STA 29+35 | CMP | 18" | 2.4 | 10 | < 10 | NA | Entrance damaged, discharge end plugged with sediment |  |
| Bluebird Lane | CV-C23-19 STA 16+60 STA 17+30 | CMP | 2 - 24" | 3.7 | 44 | 65 | 0.52 | North culvert entrance damaged, both ends 2 silted in | $ HW/D > 1.5 |
|  | CV-C23-26 STA 23+15 | RCP | 42" | 5.0 | 82 | 42 | NA | Entrance clear, discharge end 2 blocked with sediment |  |
|  | CV-C23-29 STA 19+15 | CMP | 24" | 3.0 | 18 | 15 | NA | Entrance damaged |  |
|  | CV-C23-38 STA 13+05 | CMP | 18" | 3.3 | 12 | 44 | 2.1 | Brushy approach | $ HW/D > 1.5 |
| Cedar Hill Place | CV-C23-43 STA 13+70 | RCP | 24"  18" | 2.3 | 16 | 18 | 0.28 | Rock and brush in approach channel |  |
| Cougar Loop | CV-C23-27 STA 33+55 | RCP | 24" | 2.8 | 21 | < 10 | NA | Light sedimentation at entrance |  |
|  | CV-C23-28 STA 36+50 | RCP | 24" | 3.4 | 26 | 34 | 0.13 | Entrance a silted in, brush in approach channel | $ HW/D > 1.5 |
| Juniper Hill Road | CV-C23-01 STA 39+85 | RCP | 2 - 36" | 4.2 | 110 | 718 | 8.0 | Entrance 2 silted in | $V  d > 6 $d > 0.67' |
|  | CV-C23-05 STA 55+40 | RCP | 30" | 2.9 | 30 | 400 | Avulsion (see maps) | Light brush at entrance | $V  d > 6 $Avulsion $d > 0.67' |
|  | CV-C23-11 STA 68+55 | RCP | 5 - 30" | 3.7 | 200 | 908 | Avulsion (see maps) | Brush in approach channel, barrels clear | $Avulsion |
|  | CV-C23-16 STA 73+90 | RCP | 24" | 2.9 | 22 | 29 | Avulsion (see maps) | Barrel clear | $Avulsion |
|  | CV-C23-18 STA 49+20 | RCP | 24" | 4.1 | 28 | 77 | Avulsion (see maps) | Brush at entrance | $ HW/D > 1.5 $Avulsion |
|  | CV-C23-45 STA 21+40 | RCP | 24" | 5.5 | 35 | 32 | NA(2) | Entrance and discharge ends blocked with debris |  |
|  | CV-C23-46 STA 19+30 | RCP | 42" | 4.5 | 80 | 158 | Avulsion (see maps) | Entrance 3 silted in | $Avulsion |
|  | CV-C23-49 STA 29+80 | RCP | 30" | 3.3 | 35 | 13 | NA | Entrance obstructed with brush and 2 silted in |  |
| Live Oak Loop | CV-C23-25 STA 21+75 | RCP | 2 - 42" 2 - 48" | 5.5 5.7 | 184 245 429 Total | 989 | Avulsion (see maps) | Barrels 3 - a silted in | $ HW/D > 1.5 $Avulsion |
| Live Oak Place | CV-C23-32 STA 10+50 | RCP | 24" | 3.0 | 22 | 65 | 0.72 | Brush in approach channel |  |
|  | CV-C23-33 STA 14+00 | RCP | 24" | 3.0 | 22 | 65 | 2.6 | 2 to : silted in, heavy brush at discharge end | $d > 0.67' $Does not restrict 100-yr access |
| Live Oak Road | CV-C23-56 STA 14+75 | RCP | 24" | 1.4 | 21 | < 10 | NA | Light brush and sediment in approach and discharge end |  |
| Roadrunner Lane | CV-C23-15 STA 13+50 | CMP | 2 - 24" | 2.8 | 34 | 147 | Avulsion (see maps) | Entrance 2 silted in, heavy brush in approach channel | $Avulsion |
|  | CV-C23-40 STA 15+55 | CMP | 1 - 24" 1 - 18" | 2.6 1.7 | 16 7 23 Total | 59 | Avulsion (see maps) | Both ends of 18" pipe damaged, barrels silted in 3 to a of pipe diameter | $Avulsion |
| Tramway Lane | CV-C23-14 STA 31+15 | RCP | 2 - 24" | 2.6 | 40 | 218 | Avulsion (see maps) | Brush in approach channel, 2-3" sedimentation in barrels | $Avulsion |
|  | CV-C23-22 STA 56+00 | RCP | 24" | 2.9 | 21 | 104 | 1.4 | Brushy approach channel, 3 silted in |  |
|  | CV-C23-23 STA 54+20 | RCP | 24" | 3.6 | 26 | 63 | 0.8 potential avulsion(3) | Light brush in approaches discharge channels, barrel a silted in | $ HW/D > 1.5 $Avulsion |
|  | CV-C23-24 STA 43+50 | CMP RCP | 2 - 36"  24" 1 - 24" | 2.7 3.5 | 55 25 80 Total | 1195 | 8.4(2) | Planned upgrade replaces culverts with 3 - 10'  6' box culverts | $V d > 6 $d > 0.67' $AMAFCA Project Underway |
|  | CV-C23-41 STA 18+35 | RCP | 12" | 2.7 | 6 | < 10 | NA | Silted in, brushy approach channel |  |
| Tramway Lane | CV-C23-42 STA 21+80 | RCP | 24" | 3.4 | 25 | 16 | NA | Entrance a silted in, brush in approach channel |  |
| White Oaks Drive | CV-C23-17 STA 23+20 | RCP | 4 - 48" | 5.2 | 440 | 971 | 6.3 potential avulsion(3) | Barrels clear, light brush in approach channel | $V d > 6 $d > 0.67' |
|  | CV-C23-36 STA 13+95 | RCP | 18" | 3.3 | 15 | 24 | 0.21 | Light brush at entrance, discharge end plugged with debris | $ HW/D > 1.5 |
|  | CV-C23-55 STA. 24+40 | RCP | 15" | 2.3 | 8 | 7 | NA | Discharge end obstructed by rocks and fill |  |
| Whitetail Road | CV-C23-06 STA 12+45 | RCP | 30" | 3.5 | 35 | 504 | 5.8 potential avulsion(3) | Brush in approach | $Avulsion $d > 0.67' |
|  | CV-C23-37 STA 19+70 | RCP | 18" | 4.3 | 14 | 8 | NA | Entrance and discharge ends a silted in, light brush in approach channel |  |
| Zone Atlas Page D-21 |  |  |  |  |  |  |  |  |  |
| San Antonio Drive | CV-D21-01 STA 24+00 | CMP | 24" | 1.0 | 10 | 4 | NA | Silted inlet |  |
| Zone Atlas Page D-22 |  |  |  |  |  |  |  |  |  |
| San Antonio Drive | CV-D22-01 STA 88+2-0 | CMP | 24" | 3.0 | 18 | 4 | NA |  |  |
| Zone Atlas D-23 |  |  |  |  |  |  |  |  |  |
| Antelope Avenue | CV-D23-02 STA 27+85 | RCP | 18" | 2.9 | 12 | 192 | 2.9 Potential avulsion(2) | Entrance b pugged with rocks, discharge end 2 to a plugged with debris, heavy brush in approach channel | $ HW/D > 1.5 $Avulsion $ d > 0.67' |
|  | CV-D23-37 STA 17+95 | RCP | 18" | 2.4 | 9 | 50 | 0.61 | Discharge end a silted in |  |
| Big Horn Ridge Road | CV-D23-38 STA 34+70 | RCP | 15" | 3.0 | 7 | 34 | 1.1 | Entrance 2 silted in |  |
|  | CV-D23-45 STA 43+10 | RCP | 15" | 3.2 | 11 | 40 | Avulsion (see maps) | Entrance 2 filled with sediment, large rock in approach channel | $Avulsion $ HW/D > 1.5 |
| Bluebell Drive | CV-D23-26 STA 15+25 | RCP | 18" | 2.4 | 9 | 9 | NA | Heavy brush at discharge end |  |
|  | CV-D23-27 STA 12+35 | RCP | 24" | 2.9 | 21 | 19 | NA | Heavy brush at discharge end |  |
| Bobcat Boulevard | CV-D23-13 STA 30+40 | RCP | 18" | 2.1 | 10 | 72 | 1.6 | Light brush in approach |  |
|  | CV-D23-24 STA 14+60 | RCP | 24" | 2.4 | 18 | 19 | 0.17 | Light brush in approach |  |
|  | CV-D23-25 STA 14+00 | RCP | 24" | 3.1 | 22 | 29 | 0.17 | Heavy brush at entrance, barrel and discharge end 2 silted in |  |
|  | CV-D23-43 STA 17+60 | RCP | 15" | 1.8 | 7 | < 10 | NA | Entrance and discharge ends 2 to a silted in |  |
| Eagle Ridge Drive | CV-D23-47 STA 11+80 | CMP | 48" | 5.8 | 100 | 97 | NA | Light sediment in invert |  |
| Eagle Ridge Lane | CV-D23-50 STA 13+50 | CMP | 18" | 1.8 | 14 | 24 | 0.39 | Rocks in entrance |  |
|  | CV-D23-51 STA 10+80 | CMP | 18" | 1.4 | 11 | < 10 | NA | Light brush in approach |  |
|  | CV-D23-52 STA 14+70 | CMP | 2 - 36" | 1.4 | 96 | 119 | 1.4 | Approach clear |  |
| Eagle Ridge Place | CV-D23-48 STA 13+20 | CMP | 24" | 3.0 | 18 | 41 | 2.7 | Light sediment in invert |  |
|  | CV-D23-49 STA 15+30 | CMP | 2 - 30" | 3.1 | 52 | 84 | 1.5 | Light brush in entrance |  |
| Goldenrod Drive | CV-D23-35 STA 13+20 | RCP | 24" | 3.6 | 27 | 27 | NA | Each end 3 to 2 silted in |  |
| Honeysuckle Drive | CV-D23-39 STA 21+35 | RCP | 18" | 2.9 | 10 | < 10 | NA | Entrance invert partially obstructed with concrete waste |  |
| Laurel Loop | CV-D23-23 STA 32+10 | RCP | 24" | 3.7 | 26 | 71 | 1.3 | Heavy brush at entrance | $ HW/D > 1.5 |
|  | CV-D23-30 STA 16+10 | RCP | 24" | 3.4 | 25 | 33 | 0.66 | Light brush at entrance | $ HW/D > 1.5 |
|  | CV-D23-31 STA 14+25 | RCP | 18" | 3.4 | 12 | 14 | 0.10 | Brush at entrance, discharge end 2 silted in | $ HW/D > 1.5 |
|  | CV-D23-32 STA 37+25 | RCP | 24" | 2.6 | 19 | 22 | 0.50 | Approach clear |  |
|  | CV-D23-41 STA 30+45 | RCP | 24" | 3.0 | 22 | < 10 | NA | Light brush at entrance and approach channel |  |
| Lynx Loop | CV-D23-04 STA 12+40 | RCP | 18" | 1.8 | 9 | 147 | 3.0 | Entrance 2 blocked with sediment and debris | $ d > 0.67' |
| Marigold Drive | CV-D23-21 STA 21+25 | RCP | 24" | 2.5 | 19 | 47 | 0.83 | Light brush at entrance, a silted in at discharge end |  |
|  | CV-D23-34 STA 12+60 | RCP | 42" | 5.1 | 90 | 50 | NA | Light brush in approach channel |  |
| Quail Run Court | CV-D23-29 STA 12+10 | RCP | 15"(2) | 2.8 | 9 | 17 | 0.50(2) | Entrance obstructed by brush and landscaping debris | $ HW/D > 1.5 |
| Quail Run Drive | CV-D23-03 STA 13+90 | CMP | 42" | 4.5 | 65 | 83 | Avulsion (see maps) | Brush in approach channel | $Avulsion flows in easement |
|  | CV-D23-44 STA 19+40 | CMP | 42"  30" | 3.2 | 34 | 76 | Avulsion (see maps) | Approach clear | $Avulsion flows in easement |
|  | CV-D23-46 STA 13+90 | CMP | 42" | 4.1 | 60 | 118 | Avulsion (see maps) | Heavy debris blocking approach and entrance | $Avulsion flows in easement |
| Quail Run Loop | CV-D23-20 STA 13+85 | CMP | 42"  30" | 4.0 | 40 | 59 | Avulsion (see maps) | Approach channel clear | $ HW/D > 1.5 $Avulsion |
| Red Oaks Loop | CV-D23-06 STA 16+85 | RCP | 24" | 3.0 | 22 | 106 | 2.1 | Approach channel clear |  |
|  | CV-D23-07 STA 25+45 | RCP | 24" | 2.6 | 17 | 97 | Avulsion (see maps) | Approach channel clear | $Avulsion |
|  | CV-D23-15 STA 13+80 | RCP | 24" | 2.5 | 17 | 44 | 0.66 | Light Brush in approach |  |
|  | CV-D23-16 STA 27+95 | RCP | 24" | 3.6 | 27 | 11 | NA | Barrel clear, discharge end a silted in |  |
|  | CV-D23-18 STA 34+70 | RCP | 24" | 2.3 | 15 | < 10 | NA | Approach channel clear |  |
| Tramway Lane | CV-D23-01 STA 83+95 | RCP | 18" | 2.9 | 14 | 221 | 2.18 | Approach channel clear | $ HW/D > 1.5 |
|  | CV-D23-08 STA 137+00 | RCP | 24" | 2.3 | 16 | 89 | 0.61 | Discharge end 2 silted in, rocks and brush in discharge channel |  |
|  | CV-D23-14 STA 108+45 | RCP | 24" | 4.1 | 28 | 62 | 0.32 | Large rocks in approach channel | $ HW/D > 1.5 |
|  | CV-D23-17 STA 113+20 | RCP | 24" | 2.8 | 21 | 43 | 0.25 | Heavy brush in approach channel |  |
|  | CV-D23-19 STA 129+80 | RCP | 24" | 3.0 | 22 | < 10 | NA | Approach channel partially obstructed with rocks |  |
|  | CV-D23-36 STA 133+05 | RCP | 24" | 2.5 | 18 | < 10 | NA | Discharge end 2 silted in and partially obstructed by rocks and brush |  |
| White Oaks Drive | CV-D23-42 STA 33+50 | RCP | 15" | 2.2 | 8 | 53 | Avulsion (see map) | Barrel a silted in | $ HW/D > 1.5 $Avulsion |

| BERNALILLO COUNTY DRAINAGE MASTER PLAN TABLE B - DIP SECTION CROSSINGS SANDIA HEIGHTS SOUTH |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROAD NAME | LOCATION NUMBER | 100-YEAR DISCHARGE RATE (cfs) | VELOCITY x DEPTH (fps x ft) | COMMENTS |
| Zone Atlas Page C-23 |  |  |  |  |  |  |  |  |
| Black Bear Road | DS-C23-10 STA 44+00 | 26 | 2.62  0.26 = 0.68 |  |
| Cougar Loop | DS-C23-01 STA 15+00 | 290 | 4.91  1.05 = 5.16 | $ d > 0.67' |
|  | DS-C23-02 STA 25+50 | 274 | Avulsion | $ Avulsion |
| Juniper Hill Road | DS-C23-07 STA 43+00 | 14 | 2.30  0.18 = 0.41 |  |
|  | DS-C23-19 STA 25+00 | 29 | Avulsion | $ Avulsion |
| Live Oak Court | DS-C23-13 STA 19+00 | 996 | 6.72  2.29 = 15.4 | North Domingo Baca Diversion to South Domingo Baca Dam $ V  d > 6 $ d > 0.67' |
|  | DS-C23-14 STA 17+00 | 996 | 7.18  2.57 = 18.4 | North Domingo Baca Diversion to South Domingo Baca Dam $ V  d > 6 $ d > 0.67' $Improvement not required for 100-yr storm event access if DS-C23-13 is upgraded |
| Live Oak Loop | DS-C23-09 STA 11+00 | 40 | 3.03  0.47 = 1.42 |  |
|  | DS-C23-15 STA 35+00 | 979 | 6.51  2.46 = 16.0 | North Domingo Baca Diversion to South Domingo Baca Dam $ V  d > 6 $ d > 0.67' |
|  | DS-C23-16 STA 31+50 | 10 | 1.93  0.12 = 0.23 |  |
| Pinon Hill Place | DS-C23-05 STA 24+35 | 640 | Avulsion | North Branch North Domingo Baca Arroyo $ Avulsion |
|  | DS-C23-06 STA 12+40 | 640 | Avulsion | North Branch North Domingo Baca Arroyo $ Avulsion |
| Roadrunner Lane | DS-C23-04 STA 22+50 | 39 | 2.89  0.52 = 1.5 |  |
| Tramway Lane | DS-C23-03 STA 27+00 | 13 | 2.44  0.37 = 0.90 |  |
|  | DS-C23-08 STA 62+00 | 100 | 3.72  0.84 = 3.1 | $ d > 0.67' |
|  | DS-C23-12 STA 67+50 | 1002 | 5.22  0.92 = 4.8 | North Domingo Baca Diversion to South Domingo Baca Dam $ d > 0.67' |
|  | DS-C23-18 STA 37+00 | 48 | 2.31  0.17 = 0.39 |  |
| Whiteoaks Drive | DS-C23-11 STA 17+00 | 12 | 2.16  0.29 = 0.63 |  |
| Zone Atlas Page D-23 |  |  |  |  |  |  |  |  |
| Bobcat Boulevard | DS-D23-05 STA. 19+00 | 10 | 2.30  0.33 = 0.76 |  |
|  | DS-D23-12 STA. 23+50 | 2285 | 19.6  2.22 = 43.6 | South Domingo Baca Arroyo $ d > 0.67' $ V  d > 6.5 |
| Eagle Ridge Drive | DS-D23-14 STA 15+00 | 34 | 2.94  0.54 = 1.59 |  |
| Honeysuckle Drive | DS-D23-19 STA 13+75 | 375 | 5.13  1.56 = 8.0 | V  d > 6.5 d > 0.67' |
| Laurel Loop | DS-D23-04 STA. 20+00 | 91 | 3.50  0.76 = 2.66 | $ d > 0.67' |
|  | DS-D23-13 STA 28+50 | 24 | 2.84  0.50 = 1.42 |  |
| Lynx Loop | DS-D23-01 STA. 20+00 | 45 | 3.24  0.54 = 1.75 |  |
|  | DS-D23-02 STA. 29+00 | 40 | 2.70  0.41 = 1.11 |  |
|  | DS-D23-11 STA. 36+00 | 115 | 5.11  0.30 = 1.53 |  |
| Marigold Drive | DS-D23-03 STA. 19+50(N) | 97 | 2.97  0.48 = 1.43 |  |
|  | DS-D23-07 STA. 14+50(N) | 61 | 2.97  0.55 = 1.63 |  |
|  | DS-D23-16 STA. 25+50(S) | 4245 | 11.0  4.71 = 51.8 | V  d > 6.5 d > 0.67' |
| Morning Glory Place | DS-D23-06 STA. 19+20 | 36 | 1.81  0.11 = 0.20 |  |
| Paintbrush Drive | DS-D23-18 STA 14+35 | 47 | 2.67  0.44 = 1.2 |  |
| Quail Run Court | DS-D23-08 STA. 14+00 | 92 | 2.98  0.36 = 1.1 |  |
|  | DS-D23-10 STA. 19+00 | 16 | 2.18  0.16 = 0.35 |  |
| Rockrose Road | DS-D23-09 STA. 12+25 | 17 | 2.98  0.36 = 1.07 |  |