NORTH ALBUQUERQUE ACRES

AND

SANDIA HEIGHTS-SOUTH

DRAINAGE STUDY

INFRASTRUCTURE PLAN - PHASE II

VOLUME I

Prepared For:

Bernalillo County Public Works Division

Prepared By:

ENGINEERS AND ENVIRONMENTAL SCIENTISTS

1720-B Randolph Rd. SE, Albuquerque, NM  87106

Telephone (505) 243-7300

Fax (505) 243-7400

rti@nmia.com

RTI Project No. 97-050

November  1999

VOLUME I

TABLE OF CONTENTS

I.	INTRODUCTION	1

A.	General	1

B.	Study Area	2

C.	Hydrology	2

II.	FLOOD PRONE AREA MAPS	4

A.	General	4

B.	Hydraulic Analysis Methodology	4

C.	Map Information - Erosion Setbacks	5

III.	EXISTING FACILITIES	6

A.	General	6

B.	Culvert Crossing and Dip Section Criteria	6

C.	Structure Evaluation Methodology	7

D.	North Albuquerque Acres B Phase II Existing Facilities	8

IV.	DRAINAGE INFRASTRUCTURE PLAN FOR NORTH ALBUQUERQUE

ACRES - PHASE II	11

A.	Criteria	11

B.	Facilities Proposed as Future Public and Private Sector Projects	12

C.	Major Elements of Phase II of the North Albuquerque Acres

Infrastructure Plan	13

V.	PRIORITIZATION OF INFRASTRUCTURE REQUIREMENTS	24

A.	General	24

B.	Scoring System	24

C.	Results	24

VI.	REFERENCES	30

Tables

Table 1	Summary of Manning=s n Values	4

Table 2	North Albuquerque Acres Culvert Crossings - Phase II	9

Table 3	North Albuquerque Acres Proposed Drainage Structures - Phase II	15

Table 4	Weighted Point System	24

Table 5	North Albuquerque Acres Proposed Drainage Structures Ranked by Priority - Phase II	26

Figures

Figure 1	Vicinity Map 		3

Figure 2	Proposed Storm Drains Between Ventura St. and Eubank Blvd.	20

List of Appendices

Cost Estimates

Miscellaneous Cost Alternatives

Miscellaneous Calculations

- INTRODUCTION

  - General

The Infrastructure Plan for the North Albuquerque Acres and Sandia Heights-South Drainage Study is intended to provide guidance for future drainage infrastructure development in the study area and to assist both private engineering consultants and County Development Review staff in designing safe and functional home sites within the existing North Albuquerque Acres and Sandia Heights-South subdivisions.  This report documents the Phase II portion of the study which includes drainage improvements for the unincorporated portion of North Albuquerque Acres west of Eubank Boulevard.  The Phase I area includes Sandia Heights South and the unincorporated portion of North Albuquerque Acres east of Eubank Boulevard.  Figure 1 shows both study areas.  The June 1999 North Albuquerque Acres and Sandia Heights South Drainage Study Infrastructure Plan - Phase I  Report (RTI, 1999a) documents the Phase I infrastructure plan.

The Phase II infrastructure plan is presented in three volumes.  This report is the first volume with its associated appendices that include supporting information for the engineering analyses and cost estimates.  Volume II includes Flood Prone Area Maps and Drainage Infrastructure Maps shown by Zone Atlas Page.  Volume III provides Plan and Profile sheets that show proposed structures and alignments for roads in the study area.

Infrastructure recommendations are included in this plan to bring arroyo road crossings up to current county standards, to maintain historic arroyo drainage patterns and to control avulsions at road crossings.  Avulsions are locations where all or part of an arroyo flow can take an alternate path.  The AMAFCA La Cueva, El Camino and North Camino Arroyos Drainage Facility Plan (RTI, 1999b) contains a comprehensive discussion of the avulsion problem for the major arroyos in the area.  A number of avulsions have been identified in the North Albuquerque Acres area with the eight most significant sites identified as Avulsions #1 through #8.  Avulsions #5, #7, and #8 are located in the Phase II area.  Avulsion #6 is located west of the Phase II area near Glendale and Louisiana.  The infrastructure recommendations in this report are made with the assumption that upstream avulsions have been controlled.  Specifically, Avulsions #1, #2, #3, and #4 are assumed to be controlled.  In addition to major avulsions identified in the AMAFCA report, a number of minor avulsions have occurred at arroyo road crossings in the study area.  These minor avulsions were examined in this study.

An additional objective of the Drainage Study is to identify areas within North Albuquerque Acres where conscientious development on the part of home builders can take place without the requirement for a site-specific Grading and Drainage Plan prepared by a Registered Professional Engineer.  A separate report has been prepared to discuss the criteria for these locations and to identify requirements for waiving the requirement for an Engineer-prepared Grading and Drainage Plan.  That report is under separate cover entitled the North Albuquerque Acres Site Grading and Drainage Report (RTI, 1999c).  The Site Grading and Drainage Report also provides guidance for stormwater conveyance and erosion control features that can be adapted to specific lots in the study area.  Maps have been prepared for both phases of the study that show the location of individual lots that are eligible for waiver of the requirement for an Engineer-prepared Grading and Drainage Plan.

  - Study Area

The Phase II study area is bounded on the north by the Sandia Indian Reservation, on the east by Eubank Boulevard, on the west by the Municipal Limit (City of Albuquerque), and on the south by Paseo Del Norte.  The area west of Eubank Blvd. is made up of the North Albuquerque Acres (NAA) Subdivision, platted in the 1930's.  NAA is predominantly laid out with a rectangular street grid with one-acre lots.  Minor portions of the Phase II area were replatted in the early 1990's as the Ventura Estates, Del Arroyo, Oakland Heights, and San Diego Court subdivisions, and a 26-acre sports facility was developed at the extreme western end of the study area.  Overall, NAA underwent little development prior to the late 1980's.  In the last ten years approximately one-third of the one-acre lots, excluding several large tracts owned by AMAFCA and used for flood control facilities, have been developed.

  - Hydrology

The hydrological analysis of the study area (Phases I and II) is presented under separate cover entitled Hydrology Report for the North Albuquerque Acres and Sandia Heights-South Master Drainage Plan (RTI, 1998a).  It is based on related studies performed by Resource Technology, Inc. for the AMAFCA La Cueva, El Camino and North Camino Drainage Facility Plan (RTI, 1999b) and the FEMA Re-Study of the North and South Domingo Baca and Pino Arroyos (RTI, 1996).  It is also consistent with the downstream hydrology for the City of Albuquerque=s Final North Albuquerque Acres Master Drainage Plan (RTI, 1998b).

INSERT FIGURE 1

- FLOOD PRONE AREA MAPS

  - General

A hydraulic evaluation of major and minor arroyos in the study area was conducted on arroyos with future developed condition 100-year flow rates (Q100) greater than 10 cfs.  The analyses were performed to determine the need for hydraulic structures, erosion setback distances and energy grade line (EGL) elevations for arroyos not currently shown on Federal Emergency Management Agency (FEMA) Flood Insurance Study (FIS) mapping.  Flood Prone Area Maps were prepared to show EGL elevations and erosion setbacks based on these analyses.  The Flood Prone Area Maps provide standardized, specific information about required erosion setbacks and floodplain information on a lot-by-lot basis.  The Flood Prone Area Maps are included in Volume II, Sheets 2 through 12.

  - Hydraulic Analysis Methodology

- 1.	Minor Arroyos

The Energy Grade Line elevations for minor arroyos (Q100 between 10 and 100 cfs) were calculated using Manning=s Equation.  Representative cross sections were developed to characterize reaches of the arroyo using two-foot contour interval topographic mapping (T. Mann & Assoc., 1994).  Manning=s An@ values used in these analyses are listed in Table 1.

EGL heights (depths) were developed for individual minor arroyo reaches with calculations made at arroyo junctions (where flow rates increase) or at significant changes in arroyo cross section.  EGL heights reported on the Flood Prone Area Maps are referenced to the lowest point in the arroyo.  Linear interpolation can be used to determine the appropriate EGL heights at intermediate points.

Erosion setback (ESB) distances for minor arroyos were designated to be 25 feet on each side of the arroyo flow line.

| TABLE 1 SUMMARY OF MANNING=S n VALUES |  |  |
| --- | --- | --- |
| Natural Arroyo with Rock and Vegetation | 0.035 | Engineering Judgement |
| Earth Bottom Channel | 0.030 | (COA DPM, 1997) |
| Asphalt Roadways | 0.017 | (COA DPM, 1997) |

- Major Arroyos

Water surface profiles for major arroyos (Q100  > 100 cfs) were calculated using the U.S.A.C.E.  HEC-RAS computer program.  Future developed condition discharge rates from the 1998 Hydrology Report for the North Albuquerque Acres/Sandia Heights Master Drainage Plan (RTI, 1998a) were used in the hydraulic analysis.  Arroyo cross section geometry was developed from two-foot contour interval topographic mapping (T. Mann and Assoc., 1994).  Cross sections were cut at 100-foot intervals along the arroyo centerline and Manning=s n values used in the analysis are given in Table 1.

The Energy Grade Line elevations shown on the flood prone area maps were based on a supercritical flow regime analysis almost exclusively.  Exceptions to supercritical flow calculations were made at high roadway fill cross sections where a backwater could be expected; at those locations the HEC-RAS mixed flow regime option was used to establish a profile.  Results of HEC-RAS water surface profile calculations are provided in the Appendices.

EGL elevations are reported by arroyo centerline station in tabular form on the  Flood Prone Area Maps.  The maps also show the corresponding alignments and station (cross-section) locations.  Flow velocity and depth at cross sections located at existing dip sections were checked to determine if BCPWD criteria were met (as described in Section IV of this report).

Erosion setback distances (ESB) for arroyos with 100 cfs < Q100 <200 cfs were calculated using the following equation:

ESB = (0.30) Q100

The above equation was developed during this study to provide for a smooth transition from the minimum erosion setback distance of 25 feet used for minor arroyos and the more theoretically rigorous methodology developed for AMAFCA in the Sediment and Erosion Design Guide (RCE, 1994) that is applied to major arroyos.

Erosion setbacks for arroyos with Q100 > 200 cfs were calculated using the AMAFCA Sediment and Erosion Design Guide methodology for center line setbacks.

  - Map Information - Erosion Setbacks

The flood prone area maps show future developed condition 100-year flow rates, EGL elevations, erosion setbacks, FIS floodplains, and locations of major and minor avulsions.  EGL elevations should be used to establish minimum pad elevations.  Erosion setback distances are controls on encroachments into arroyo floodplains.  The primary structure on a given property must be located no closer to the low point of the arroyo flow line than the erosion setback distance indicated on the Flood Prone Area Maps.  Where the 100-year flow rate is greater than 10 cfs, drainage easements corresponding to the erosion setback line must be granted to the County at the time when a building permit is issued.  It is recommended that the easement be shown as one or more tangent lines with the maximum deviation from the true erosion setback line being no more than one-quarter of the erosion setback distance.  However, all parts of the primary structure should be located more than the calculated ESB distance from the low point of the arroyo flow line.

- EXISTING FACILITIES

  - General

An inventory of existing road crossing drainage structures and storm drain facilities was conducted in the study area.  Road crossings included culverts and dip sections.  Field observations and culvert capacity calculations were used to determine if road crossings are acceptable when conveying the future developed condition 100-year discharge.  Road crossings of arroyos with 100-year peak discharge rates greater than 10 cfs were evaluated.

The inventory included arroyo crossings of collector and local streets in the study area but did not include driveway culverts.  Drainage structures with deficiencies were identified and a proposed remedy was developed.  This section describes the criteria and methods used in the structure inventory and roadway crossing evaluation and summarizes the results of the work.

  - Culvert Crossing and Dip Section Criteria

Deficiency criteria for crossing locations were developed based on the potential to cause injury and/or property damage when conveying the 100-year peak discharge flow.  Crossing deficiencies used in the evaluation are:

- 1.	Avulsion

The culvert or dip section does not have the capacity to convey the 100-year peak discharge rate and the excess flow leaves the arroyo channel and seeks a different path.  The diversion of flow results in flooding and erosion and in some cases has the potential to damage property and cause personal injury.

- Velocity  Depth > 6.5 for Flow Crossing the Roadway (V  d > 6.5)

Crossings were classified under this deficiency if the product of flow velocity times flow depth (V  d) for the excess flow over the roadway is greater than 6.5.  This deficiency acknowledges the public safety concern associated with roadway overtopping because a vehicle crossing a flow under these conditions could potentially be swept downstream.

- 3.	Depth of Flow Over Roadway > 0.67 ft. (d > 0.67 ft.)

A crossing has this deficiency if it is unable to convey the 100-year peak discharge rate and depth of flow over the roadway is greater than 0.67 ft.

- 4.	Headwater Height to Diameter Ratio > 1.5 (HW/D > 1.5, culverts only)

This criterion compares the maximum depth of water at a culvert entrance (headwater) to the diameter of the culvert pipe.  If the required headwater depth is too great, sedimentation will occur as the water ponds at the approach to the culvert entrance.  Sedimentation can occur to the point that the culvert is obstructed and flow will overtop the roadway.  Culverts that had a ratio of headwater to diameter greater than 1.5 were considered deficient.

- Backwater flooding (when criteria 1 through 4 are met, culverts only)

Culvert crossings were examined for flooding potential from water that backs up behind culvert entrances.  A culvert was found to be deficient if the potential for property damage from backwater flooding that is distinct from avulsion or roadway overtopping could exist.

  - Structure Evaluation Methodology

- 1.	Field Survey of Existing Structures

Culvert and dip section crossings were identified using aerial photos, topographic mapping, and field investigation.  Existing culvert crossing locations are shown on the Flood Prone Area Maps.  Existing culvert locations in NAA are noted by a number beginning with CV followed by the Zone Atlas page number and an individual identification number.

Culvert crossing locations and dip sections were inspected in the field.  The type, size, and number of pipes was documented and the maximum freeboard available was measured.  The condition of the culvert and the arroyo channel was noted for each location.  Crossings were examined to identify locations where flooding could occur from potential backwater conditions.  Table 2 lists the existing culvert crossings and their locations.  Dip sections were field checked to rule out or confirm the existence of an obstructed or abandoned culvert.  Existing storm drain facilities were not identified in Phase II.  Storm drains proposed for the planned Paseo Del Norte improvements were not included in this analysis.

  - Culvert and Dip Section Capacity Calculations

The capacity of each culvert location was determined using the maximum headwater available at the culvert(s) as determined by field measurement.  Capacity calculations were made using engineering nomographs for inlet control conditions (FHWA, 1985).  Sediment accumulated at the culvert entrance was ignored in making the maximum capacity calculations.  Table 2 lists the maximum capacity at each culvert along with the 100-year discharge rate for that location.  For cases where the 100-year discharge rate exceeded the culvert capacity, the roadway elevation and alignment were examined to determine if overtopping would occur or if an avulsion would result.

The depth of flow over the roadway was calculated where roadways were overtopped and the flow rejoined the flow path on the other side of the roadway.  Overtopping flow depths for total arroyo flows less than 100 cfs were calculated with Manning=s equation using a cross-section developed from topographic mapping of the location.  Water surface profiles using HEC-RAS were developed for arroyos where the 100-year discharge rate exceeded 100 cfs.  Roadway flow depths were determined from HEC-RAS results at road crossing cross-sections.

Flow depths and velocities for dip sections with flow rates less than 100 cfs were calculated using Manning=s equation for a cross-section at the roadway centerline developed from topographic mapping.  HEC-RAS water surface profile calculations were used to determine flow depth and velocity through dip sections with flow rates greater than 100 cfs.

  - North Albuquerque Acres B Phase II Existing Facilities

- 1.	Culvert Inventory

Five existing culvert crossings were inventoried in the Phase II area of North Albuquerque Acres.  Two locations were found deficient when conveying the 100-year peak discharge.  One of the existing deficient crossings results in an avulsion of approximately 32 cfs along Signal Ave. (CV-C21-01).  Structures proposed upstream of this facility, if implemented, will lower the flow rate at its location to a level within its capacity.

The culvert crossing for a tributary to the North Domingo Baca system at Ventura Street (CV-C20-01) has a ratio of headwater to pipe diameter (HW/D) of 1.9 (> 1.5).  Because this crossing is in the right-of-way for the incorporated portion of North Albuquerque Acres and does not result in an avulsion or residential flooding, an improvement was not listed in this study.

Table 2 lists culvert locations and associated deficiencies. Proposed improvements are discussed in Section IV.

| TABLE 2 NORTH ALBUQUERQUE ACRES CULVERT CROSSINGS B PHASE II |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Road Name | Location Number | Type | Dimensions | Maximum Headwater Depth (ft) | Capacity (cfs) ** | 100-year Discharge Rate (cfs) | Velocity  Depth | Condition | Comments |
| Elena Drive | CV-B21-02 STA 127+25 | CMP | 2-24" | 2.8 | 36 | 40 | NA | Heavy sediment in approach, pipe entrances 2/3 sedimented in |  |
|  | CV-B21-03 STA 117+75 | CMP | 2-24" | 3.0 | 38 | 16 | NA | Approach and entrances clear |  |
|  | CV-B21-04 STA 106+40 | CMP | 2-24" | 2.6 | 34 | 16 | NA | Approach and entrances clear |  |
| Signal Avenue | CV-C21-01 STA 42+40 | CMP | 42" | 4.9 | 70 | 102 | Avulsion (see maps) | Approach clear, light sediment in entrance | Avulsion |
| Ventura Street* | CV-C20-01 STA 38+00 | CMP | 4-24" | 3.7 | 88 | 34 | NA | Entrances damaged, sediment and obstructions in entrances | HW/D >1.5 |

* Not in County Right-of-Way - included for information and planning by others.

** Capacity with no obstruction (sediment or debris)

- Street Review -- Dip Section Crossings

The original road grid system imposed on North Albuquerque Acres by the 1930's platting is problematic from a drainage standpoint.  As dedicated roads have been bladed, numerous minor tributaries as well as some major arroyos have been diverted along the road instead of crossing it.  As part of this drainage study road profiles were developed to maintain or help restore natural drainage patterns.  This review of road profiles included numerous locations where Adust control@ paving has been put down as well as engineered roads.  Not included were major facilities like the soon to be improved Paseo del Norte.

The majority of arroyo road crossings in the Phase II area occur in dip sections where no culvert exists.  Nearly all of the major arroyo crossings present flow depth and flow velocity problems during the 100-year storm event.

- Major Channelizations

Existing channelizations in the Phase II area include the Sunset Hills Channel on the North Camino Arroyo and two channelizations of portions of the La Cueva Arroyo that are located on the Sportsplex facility.  These channels have adequate capacity to convey 100-year future developed condition storm runoff.

AMAFCA has constructed a training dike along the La Cueva Arroyo between Holbrook and Ventura on the north side of Signal Avenue which keeps the arroyo flows from avulsion to the North Domingo Baca system at that location.  That structure was analyzed in mapping for FEMA Flood Insurance Studies.

- DRAINAGE INFRASTRUCTURE PLAN FOR NORTH ALBUQUERQUE ACRES - PHASE II

- A.	Criteria

      - General

The area west of Eubank Boulevard between Paseo Del Norte Drive and Elena Drive is characterized by a relatively uniform 3 to 4 % gradient from east to west.  The street grid that was originally imposed over it at the time of the original plat has been largely maintained even though no consideration was given to drainage patterns.  Over the years, as streets and roads have been bladed in the decomposed granite soil, numerous minor arroyo tributaries and a few major arroyos have been diverted from their original flow path to follow a new course down the road cuts roughly parallel to the natural flow patterns.

The cumulative effect of these numerous flow diversions has been a substantial redirection of the natural drainage patterns to unpredictable new flow paths following the most recently graded available alignments.  Therefore, the proposed changes to vertical alignments and horizontal alignments (within the right-of-way) of existing unpaved roads should be seen as integral to the overall drainage plan for the area.

      - Restoration and Maintenance of Natural Drainage Patterns

One of the major goals of this infrastructure plan is to restore, to the extent possible, the natural drainage patterns by the installation of sufficient road crossing structures to allow flow to cross the road Right-of-Way instead of being diverted along the Right-of-Way.  In some cases, because of existing development or structures, this was not desirable or possible and provisions had to be made for the new flow path until it rejoined a natural arroyo or an engineered flood control facility with sufficient capacity.

      - Road Crossing Criteria

Where flows cross streets and roads the proposed structures are sized to meet the previously mentioned County road crossing standards.  These include not only limitations on velocity and depth of flow but also limitations on the headwater depth in relation to culvert diameter to limit sedimentation problems.  In some instances lined or paved road side ditches are proposed when flow within the public Right-of-Way is sufficient to justify these measures.  In these areas special attention will need to be given in the future to private driveway culvert installations so as to not restrict the ditch flow capacity.

      - Local Avulsions

There are numerous locations where the natural arroyos can potentially change flow paths either over time or during a major runoff event.  Most of the major arroyo avulsions are fully discussed in the AMAFCA La Cueva, El Camino and North Camino Arroyos Drainage Facility Plan (RTI, 1999b).  As part of this study effort additional minor avulsion locations have been identified, typically these are found where one of the minor arroyos can follow alternate flow paths for a few hundred feet or more before rejoining the main (current) flow path.

In some cases these avulsions have been addressed with adjustments to road grades and culvert recommendations.  In others, facilities have been proposed on private property to effectively mitigate the situation.  Because the benefit from these measures is largely to private property owners, funding for many of these measures may be expected from the parties concerned.  In other cases, where the threat is to existing development or public facilities a more active County role could be justified.

- Facilities Proposed as Future Public and Private Sector Projects

  - General

Over the last ten years there have been several planning efforts for public and private infrastructure in the North Albuquerque Acres area.  These have resulted in proposed drainage related projects that are in various stages of planning and design.  These have been incorporated in this study to ensure a properly functioning drainage system.  The major elements of these projects are discussed below:

  - Paseo del Norte Storm Drains

Planning and design for the widening of Paseo del Norte has been in the planning stages for more than ten years.  The conceptual storm drainage plan for Paseo del Norte Drive was developed by AMAFCA in the early 1990's and is currently being implemented by the BCPWD=s consultant for the design of this major transportation facility.  The planned storm drain collects roadway drainage and runoff from adjacent properties and conveys them west in a large RCP conduit to an outfall near San Pedro Boulevard to the South Domingo Baca Arroyo system.  Because these projects are in the construction phase no further study effort was devoted to the Paseo del Norte corridor, however the drainage facilities included in the plans are shown in this study when appropriate.

- 3.	Holbrook and Carmel Storm Drains

The AMAFCA North and South Domingo Baca Arroyos and Paseo Del Norte Corridor Drainage Management Plan (RTI, 1991) proposed storm drain facilities in Holbrook from Signal Ave. to Holly and in Carmel Ave. east of Holbrook to Eubank Boulevard.  The Holbrook facility and the Carmel Storm Drain west of Holbrook were also included in the COA Final North Albuquerque Acres Master Drainage Plan (COA MDP) (RTI, 1998b).  These facilities have been incorporated into this study as follows.

The Holbrook facility picks up tributary flows in the North Domingo Baca Arroyo system west of the Upper North Domingo Baca Dam and directs them west in a storm drain in Carmel Avenue west of Holbrook.  The proposed Carmel Storm Drain picks up the discharge from the Upper North Domingo Baca Dam and minor drainages east of Holbrook and combines them with flows in the Holbrook Storm Drain.  The storm drain then directs the combined flows west to Ventura.  A channel is proposed in the COA MDP west of Ventura to convey flows from the Carmel Storm Drain.

- 4.	Eubank Boulevard and Alameda Storm Drains

BCPWD is the lead agency on the Alameda Corridor Study (Avid, 1997) which includes the proposed realignment of Alameda and the widening of Eubank from Alameda to Paseo del Norte.  Conceptual drainage design was developed for that study and that work is being coordinated between the County=s consultant on that project and this study.  Drainage features associated with the Alameda Corridor in Bernalillo County include using the Alameda Corridor roadway to confine the La Cueva Arroyo to the north side of the roadway and storm drains that collect and convey runoff from the roadways themselves.  The conceptual drainage design includes one storm drain that directs roadway drainage in the Alameda alignment to the La Cueva system at unspecified locations; a second storm drain conveys runoff from that portion of Eubank north of Paseo del Norte into the outfall for the Upper North Domingo Baca Dam.  The conceptual design described in the Avid Engineering report was expanded upon in this plan (Section IV.C.5).

- Major Elements of Phase II of the North Albuquerque Acres Infrastructure Plan

  - General

The existing drainage infrastructure for the Phase II area of North Albuquerque Acres is less developed than that in the Phase I area and Sandia Heights South.  However, of the four existing road crossing structures evaluated in County Right-of-Way only one proved to be deficient.  Numerous additional drainage structures are proposed: seven storm drain systems; one major channel and two lesser channelizations; two training dikes; forty-six road crossing culverts, four concrete box culverts, and twelve dip sections.  The road crossing culverts include ten major facilities described in more detail below.  All proposed structures, including construction cost estimates, are summarized in Table 3.  The proposed infrastructure plan emphasizes road and street alignment adjustments as well as providing for safe arroyo crossings and avulsion control.

  - Road Grades and Horizontal Alignments Adjustments

        - Road Grades

Included in the Infrastructure Plan are forty-six preliminary roadway plan and profile sheets at a scale of 1"=100' for most of the non-engineered roads between Eubank and the municipal limits.  These are presented in Volume III, Sheets 2 through 47.  However, improvements that are part of the County=s Paseo del Norte project are not included.

The plan and profile (P&P) sheets cover approximately seventeen miles of County roads and streets.  Where possible, consistent with providing reasonable sight distance and minimizing driveway relocations, the vertical alignments remain at existing grades.  Where necessary, grades were raised to accommodate proposed culverts or to maintain drainage basin separation; and in some cases grades were lowered to provide for dip sections.  Also shown on the P&P sheets are the proposed roadway culvert pipes and concrete box culverts discussed below.

        - Horizontal Alignment Adjustments

Also shown on the Plan and Profile sheets (Volume III) are minor proposed adjustments to road horizontal alignments.  These proposals are associated with channelization of a tributary to the El Camino Arroyo along Modesto Ave. west of Holbrook and the Carmel Storm Drain in the North Domingo Baca System.  These proposed structures are discussed below.

| TABLE 3 NORTH ALBUQUERQUE ACRES PROPOSED DRAINAGE STRUCTURES PHASE II BCPWD - NORTH ALBUQUERQUE ACRES/ SANDIA HEIGHTS SOUTH DRAINAGE STUDY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Structure Number | Roadway | Zone Atlas Map | Station | Structure | 100-yr Flow Rate cfs (Future) | Estimated Cost |
| 1 | ALAMEDA | C-20 | 15+30 | 1 - 36" CMP | 43 | $ 13,558 |
| 2 | ALAMEDA | C-21 | 52+53 | 2 - 10' X 8' CBC | 2770 | 143,354 |
| 3 | ALAMEDA | C-21 | EUBANK/ ALAMEDA | CHANNELIZATION | 2770 | 862,154 |
| 4 | ALAMEDA | C-20 | ALAMEDA/ VENTURA | CHANNELIZATION | 2796 | 1,561,280 |
| 5 | ALAMEDA CORRIDOR (PROPOSED) | C-21 | PASEO DEL NORTE/VENTURA | 30" TO 42" STORM DRAIN SYSTEM | VARIES | 952,349 |
| 6 | BARSTOW | B-19 | 11+80 | 3 - 48" CMP | 228 | 29,156 |
| 7 | BARSTOW | B-19 | 16+30 | 1 - 24" CMP | 19 | 7,776 |
| 8 | BARSTOW | B-19 | 21+10 | 3 - 48" CMP | 245 | 29,156 |
| 9 | BARSTOW | B-19 | 26+50 | 3 - 30" CMP | 90 | 17,529 |
| 10 | BEVERLY HILLS | B-19 | 10+10 to 21+30 | 36" STORM DRAIN | 98 | 171,344 |
| 11 | BEVERLY HILLS | B-19 | 47+30 | 3 - 36" CMP | 119 | 29,594 |
| 12 | BEVERLY HILLS | B-19 | 49+50 | 3 - 36" CMP | 119 | 29,594 |
| 13 | BEVERLY HILLS | B-19 | 63+30 | 1 - 24" CMP | 11 | 10,459 |
| 14 | BEVERLY HILLS | B-19 | 69+75 | 5 - 36" CMP | 237 | 46,624 |
| 15 | CARMEL AVE. | C-21 | 32+75 | DIP-SECTION | 10 | 21,847 |
| 16 | CORONA AVE. | C-21 | 28+50 | 1 - 36" CMP | 38 | 13,558 |
| 17 | EAGLE ROCK | C-20 | 28+73 | DIP-SECTION | 49 | 21,847 |
| 18 | EAGLE ROCK | C-21 | 42+65 | DIP-SECTION | 6 | 21,847 |
| 19 | EAGLE ROCK | C-21 | 58+19 | DIP-SECTION | 4 | 21,847 |
| 20 | ELENA DR. | B-19 | 23+50 to 32+00 LT | TRAINING DIKE | 1822 | 78,097 |
| 21 | ELENA DR. | B-19 | 26+70 | 1 - 36" CMP | 48 | 13,558 |
| 22 | ELENA DR. | B-19 | 56+44 | DIP-SECTION | 17 | 21,847 |
| 23 | ELENA DR. | B-19 | 67+00 | DIP-SECTION | 25 | 21,847 |
| 24 | ELENA DR. | C-22 | 94+40 | 1 - 24" CMP | 11 | 10,459 |
| 25 | FLORENCE | B-20 | 10+35 | 1 - 24" CMP | 8 | 10,459 |
| 26 | FLORENCE | B-20 | 12+00 | 2 - 30" CMP | 69 | 17,914 |
| 27 | FLORENCE | B-20 | 12+00 to 25+75 | 2 - PAVED SWALES | 124 & 69 | 89,445 |
| 28 | FLORENCE | B-21 | 51+50 | 1 - 24" CMP | 5 | 10,459 |
| 29 | GLENDALE | B-21 | 25+30 | 4 - 54" CMP w/EMBANKMENT | 685 | 306,786 |
| 30 | GLENDALE | B-21 | 36+80 | 1 - 24" CMP | 13 | 10,459 |
| 31 | GLENDALE | B-21 | 41+70 | 4 - 48" CMP | 646/2 | 43,470 |
| 32 | GLENDALE | B-21 | 50+50 | 3 - 48" CMP | 646/2 | 37,179 |
| 33 | GLENDALE | B-21 | 39+50 to 54+50 | 2 - ARMORED EMBANKMENT | 646/2 | 260,130 |
| 34 | HOLBROOK | C-21 | 13+70 to 39+00 | 42" TO 78" SD TO VENTURA | 685 | 1,496,974 |
| 35 | HOLBROOK | C-21 | 46+35 | 2 - 10' X 8' CBC | 2773 | 143,354 |
| 36 | HOLBROOK | C-21 | 42+60 to 50+00 | 24" TO 48" SD | 95 | 86,779 |
| 37 | HOLBROOK | C-21 | 55+40 | 1 - 30" CMP | 23 | 8,853 |
| 38 | HOLBROOK | C-21 | 59+50 | 1 - 24" CMP | 5 | 7,776 |
| 39 | HOLBROOK | C-21 | 62+20 | 2 - 36" CMP | 78 | 16,589 |
| 40 | HOLBROOK | C-21 | 63+90 | 1 - 36" CMP | 45 | 10,135 |
| 41 | HOLBROOK | B-21 | 68+80 | 2 - 10' X 4' CBC | 652 | 85,310 |
| 42 | HOLBROOK | C-21 | 73+20 | 1 - 24" CMP | 8 | 7,776 |
| 43 | HOLBROOK | B-21 | 74+70 | 2 - 36" CMP | 95 | 16,589 |
| 44 | HOLBROOK | B-21 | 80+00 | 1 - 24" CMP | 11 | 7,776 |
| 45 | MODESTO | B-20 | 13+50 | 4 - 42"CMP w/LINED CHANNEL | 253 | 220,792 |
| 46 | MODESTO | B-20 | 30+50 | 2 - 30" CMP | 58 | 13,604 |
| 47 | MODESTO | B-21 | 41+62 | DIP-SECTION | 13 | 21,847 |
| 48 | OAKLAND | C-20 | 19+69 | DIP-SECTION | 17 | 21,847 |
| 49 | OAKLAND | C-21 | 39+00 | 1 - 24" CMP | 10 | 10,459 |
| 50 | OAKLAND | C-21 | 48+50 | 1 - 24" CMP | 12 | 10,459 |
| 51 | OAKLAND | C-21 | 51+50 | 1 - 24" CMP | 14 | 10,459 |
| 52 | OAKLAND | C-21 | 56+20 | 2 - 10' X 8' CBC | 2770 | 143,354 |
| 53 | SAN DIEGO | B-19 | 22+00 to 24+30 | PAVED SWALE | 104 | 7,483 |
| 54 | SAN DIEGO | B-19 | 38+50 | 1- 24" CMP | 11 | 10,459 |
| 55 | SAN DIEGO | B-19 | 56+25 | 5 - 36" CMP | 224 | 46,624 |
| 56 | SAN DIEGO | B-20 | 63+50 | 1 - 24" CMP | 6 | 7,776 |
| 57 | SAN DIEGO | B-20 | 83+50 | 1 - 24" CMP | 22 | 10,459 |
| 58 | SAN DIEGO | B-20 | 102+09 | DIP-SECTION | 16 | 21,847 |
| 59 | SAN DIEGO | B-20 | 106+24 | DIP-SECTION | 10 | 21,847 |
| 60 | SIGNAL | C-21 | 50+60 | 1 - 30" CMP | 24 | 8,853 |
| 61 | VENICE | B-19 | 25+00 | 3 - 48" CMP w/BERM | 266 | 46,754 |
| 62 | VENICE | B-19 | 44+00 | 3 - 48" CMP | 248 | 37,179 |
| 63 | VENTURA | B-20 | 16+00 | 4 - 36" CMP | 193 | 29,716 |
| 64 | VENTURA | B-20 | 18+30 | 1 - 24" CMP | 11 | 7,776 |
| 65 | VENTURA | B-20 | 24+00 | 2 - 48" CMP | 182 | 21,211 |
| 66 | VENTURA | B-20 | 27+95 | 1 - 24" CMP | 4 | 7,776 |
| 67 | WILSHIRE | C-21 | 31+75 | DIP-SECTION | 22 | 21,847 |
| 68 | WILSHIRE | C-21 | 37+65 | DIP-SECTION | 4 | 21,847 |
| 69 | WYOMING | B-19 | 15+70 | 1 - 30" CMP | 21 | 8,853 |
| 70 | WYOMING | B-19 | 18+40 | 1 - 30" CMP | 30 | 8,853 |
| 71 | WYOMING | B-19 | 24+40 | 3 - 48" CMP | 253 | 29,156 |
| 72 | WYOMING | B-19 | 27+10 | 4 - 48" CMP | 353 | 34,361 |
| PHASE II - STRUCTURES - TOTAL COST |  |  |  |  |  | $7,688,390 |

  - Channelization

Given the low density development characteristic of the North Albuquerque Acres area channelization of the major arroyos was considered to be a measure of last resort.  Two channelization projects in the Phase II area are recommended in the AMAFCA La Cueva, El Camino and North Camino Arroyo Drainage Facilities Plan (RTI, 1999b).  These proposals include channelization of the La Cueva Arroyo west of Eubank to the Alameda Alignment and channelization of the El Camino Arroyo west of Holbrook to control Avulsion #5.

Channelization of the La Cueva Arroyo from Eubank to Alameda was proposed in the above mentioned reports.  The 300  to 500-foot wide floodplain associated with the La Cueva Arroyo at this location makes an improved conveyance of some form inevitable.  The most economical solution would be a soil cement or concrete lined channel following the general alignment shown on the P&P sheets (Volume III).  Other arroyo treatments are feasible and have been discussed in public meetings.  An alternative riprap-lined conveyance was evaluated for this study and is shown conceptually in the Appendix.  Costs are greater for the riprap-lined alternative and future maintenance costs make the concrete or soil cement lined channel more favorable.  While these other treatments might be more aesthetically acceptable or fit better into some yet to be defined open space or park scheme, such options would carry a more burdensome construction and right-of-way cost.  Therefore, unless some other entity covers the additional costs as part of a drainage improvement plan, it seems likely that a lined channel will ultimately be constructed.  The proposed La Cueva channel is included on Table 3.  The structures associated with the channel are included as road crossing structures for Oakland and Alameda.

The La Cueva Arroyo in the proposed Alameda Corridor is proposed to be conveyed to the municipal limit in a channelization or improved conveyance associated with the Alameda Corridor project.  Soil cement embankment protection has been proposed for the north side of the embankment for the Alameda Corridor roadway (AVID, 1997).  This study examined the addition of soil cement bank protection for the south side of the existing Alameda Boulevard to provide a conveyance system between the two roadways.  A series of soil cement grade control structures would be required in that improved conveyance to control headcutting in the unlined conveyance bottom.

The material and earthwork costs associated with the soil cement-lined berm conveyance exceed costs associated with a concrete or soil cement-lined channel and for that reason a lined channel was proposed in this study on the north side of the proposed Alameda Corridor.  The soil cement grade/control structure conveyance remains an alternative that can be further explored at such time that the Alameda Corridor project moves to the design phase.  The Appendix contains information about the soil cement/grade control structure option.  The channelization is included on Table 3 with a road crossing structure at Holbrook.  The flow from this channel would be picked up at the municipal limit by a hard lined channel proposed in the COA MDP (RTI, 1998b).

To control Avulsion #5 a riprap-lined channel on the north side of Glendale Avenue from Sta. 24+00 to Sta. 36+00 with four 54-inch culverts at station 25+00 is recommended.

An additional channelization is proposed in this study for a tributary to the El Camino Arroyo on the north side of Modesto Avenue.  The existing Modesto alignment is essentially in the arroyo itself and the channel is proposed to provide access to the street during major storms.  The proposed concrete-lined channel begins downstream from four 42-inch culverts that convey the flow to the north side of the roadway.  As a result a minor change in horizontal alignment for the roadway is necessary.  The channel runs from Sta. 14+50 to Sta. 20+70 with culverts placed at Sta. 20+50.  A storm drain had been investigated for this location but the channel was a lesser cost alternative.  However, the storm drain alternative is also presented in the Appendix.

  - Training Dikes

The North Camino Arroyo passes through the Phase II area on a path generally parallel to Elena Drive and south of the Sandia Reservation Boundary.  Avulsion #8 is a  major avulsion of the North Camino Arroyo located where Elena Drive intersects the municipal limits (see Flood Prone Area Maps -- Volume II).  The AMAFCA La Cueva, El Camino and North Camino Arroyo Drainage Facilities Plan (RTI, 1999b) and the City of Albuquerque Final North Albuquerque Acres Master Drainage Plan (RTI, 1998b) propose a channelization and training dike for the North Camino to control Avulsion #8.  The training dike is located on the Bernalillo County side of the municipal limits and has been included in this Plan to be consistent with the other documents.

Avulsion #7 is on a tributary to the North Camino Arroyo at Venice Avenue east of Louisiana.  The City of Albuquerque Final North Albuquerque Acres Master Drainage Plan (RTI, 1998b) proposed a dip section and training dike on Venice Avenue to control the avulsion.  The proposed  training dike has been included in this plan with the substitution of three 48-inch culverts in place of the dip section.

  - Storm Drains

While the primary focus of drainage improvements in this area was to maintain existing drainage patterns in the natural arroyos.  There are several locations where storm drains are recommended in addition to those that are being planned as part of the Paseo del Norte transportation projects.  These storm drains are shown on Figure 2 and are described below:

-	A 30"-66" storm drain in Holbrook to convey flows from various North Domingo Baca arroyo tributaries to a proposed Carmel Avenue storm drain.  Discharge from the Upper North Domingo Baca Dam and minor North Domingo Baca tributaries east of the proposed Eubank alignment is proposed to be collected in a separate 48" to 54" storm drain in Carmel to be combined with the Holbrook Storm Drain flows.  The combined flows will be conveyed west in a 72" to 78" storm drain in Carmel that extends across municipal limits to a proposed channel at Ventura.  A closed conduit to contain flows from the Upper North Domingo Baca Dam to Holbrook and the Holbrook Storm Drain was previously proposed in the AMAFCA North and South Domingo Baca Arroyos and Paseo del Norte Corridor Drainage Management Plan (RTI, 1991).

Insert Figure 2 - Proposed Storm Drains Between Eubank and Holbrook Boulevards

A system of storm drains was proposed for the Alameda Corridor project in the AVID Engineering report (1997).  The conceptual storm drain in that report collected drainage from the Alameda Corridor roadway and discharged it to the La Cueva Arroyo System on the north side of the proposed alignment.  This study expanded on the conceptual drainage design to allow the storm drains in the proposed alignment to collect flows from minor arroyos that cross the proposed alignment or from adjacent minor flows.  The location of the proposed storm drains are shown on Figure 2 and are described below.

-	ACSD-1 and ACSD-2

A 24" storm drain system is proposed by this study for the east-west Alameda Corridor alignment from Ventura Boulevard to just west of Holbrook to collect roadway flows and flow from an adjacent tributary.  This storm drain is identified as ACSD-1 on Figure 2.  This drain discharges to the proposed La Cueva Arroyo Channel east of Ventura Boulevard.  A 30" storm drain (ACSD-2) is proposed in the Alameda Corridor east of Holbrook to collect roadway flows and a former La Cueva arroyo tributary near the existing AMAFCA La Cueva Dike north of Signal Ave. This proposed storm drain runs from approximately Station 45+70 to just east of Holbrook where it discharges to the proposed La Cueva Arroyo Channel.

- 	ACSD-3 and ACSD-4

Two storm drains are proposed for the Eubank Boulevard portion of the Alameda Corridor Project.  A 36" to 42" storm drain identified on Figure 2 as ACSD-3 is proposed to collect roadway drainage and flow from minor North Domingo Baca Arroyo tributary flows that would have crossed the alignment near Wilshire and Signal Avenues.  This storm drain directs the flows to a discharge point in the proposed La Cueva Channel at Station 50+20.  A 36" to 48" system identified as ACSD-4 is recommended in the proposed Eubank Boulevard alignment to collect roadway flows and North Domingo Baca Arroyo tributary flows in the area from Wilshire Avenue to Holly east of Eubank Boulevard (Figure 2).  These flows will be directed to an outfall west of Eubank Boulevard where they will be combined with discharge from the North Domingo Baca Dam.  The combined flow is conveyed to Holbrook and Carmel Avenues by a 48" storm drain described previously.  Storm drains ACSD-3 and -4 are proposed in order to more effectively use the proposed La Cueva Channel and because interception of these tributary flows at Eubank lowers the cost of the downstream Holbrook/Carmel system.

-	ACSD-5 and ACSD-6

Two storm drains are proposed in Holbrook at the proposed La Cueva Channel crossing.  These storm drains are shown on Figure 2 as ACSD-5 and -6.  ASCD-5 is a 24" system on the south side of the proposed La Cueva Channel and ACSD-6 is a 30" to 48" system on the north side of the channel.  These drains are proposed to collect adjacent La Cueva Arroyo tributary flows and direct them to the proposed La Cueva Channel.  This study recommends the interception of these flows because the proposed channelization of the La Cueva Arroyo can be more fully utilized.

The diversion of the tributary North Domingo Baca basins intercepted by the proposed storm drains described above has minor effect on the La Cueva Arroyo system.  An increase of less than 20 cfs to the peak flow of over 2700 cfs can be expected for the 100-year storm.  AHYMO hydrologic models for the proposed diversions for the Alameda Corridor storm drains are included in the Appendix.

-	A 30" storm drain is proposed in Beverly Hills on the north side of the Sportsplex to connect to the proposed City of Albuquerque storm drain SD-1 in Venice Boulevard (RTI 1998b).  This system will collect a tributary to the North Camino Arroyo that has been diverted by lot and street grading and direct it to its historic confluence near Venice and Louisiana Boulevard.

  - Major Culverts

        - General

Where major arroyos and their tributaries cross roads or streets there are generally few alternatives to providing either multi-pipe or concrete box culverts.  Analysis of road crossing hydraulics in this study indicates that in most cases the Bernalillo County criteria for either depth < .67' or velocity times depth < 6.5 cannot be met for flows over 150 cfs crossing normal to the street R/W.  However, due to the expense of these structures proposed facilities utilize a headwater to depth ratio of up to 1.5 and, in some cases, allow up to 150 cfs of the 100-year storm runoff to flow over the road.  At locations where limited roadway overtopping was allowed to occur the flow depth times velocity criterion is met in the proposed design.  The following are the major culvert installations (concrete box culverts or four or more culvert pipes) recommended for the Phase II portion of the North Albuquerque Acres area.

        - North Camino Arroyo System

The North Camino Arroyo enters the Phase II area north of Elena Drive just west of  Eubank.  It passes through the Sunset Hills Channel north of Elena Drive to an outfall west of Holbrook where it continues to flow north of Elena.  Numerous structures are proposed on tributaries to the North Camino Arroyo which flow west and north in the area north of Glendale Avenue.

-	Beverly Hills		Sta 69+75		5-36" CMP

-	San Diego		Sta 56+25		5-36" CMP

-	Ventura		Sta 74+10		4-36" CMP

-	Wyoming		Sta 27+10		4-48" CMP

        - El Camino Arroyo

The El Camino Arroyo enters the Phase II area at Eubank between Florence and Glendale.  The arroyo generally follows the Glendale alignment to Wyoming where it heads northwest to the Sportsplex south of San Diego and east of Louisiana.  Proposed structures in the Phase II area have been sized with the assumption that the avulsion of the North Camino (Avulsion #1) near the access road to the Juan Tabo picnic area has been controlled.

-	Glendale		Sta 25+30		4-54" CMP

-	Holbrook		Sta 68+80		2-10'x4' CBC

-	Modesto		Sta 13+50		4-42" CMP (El Camino Tributary)

        - La Cueva Arroyo

The main branch of the La Cueva Arroyo enters the Phase II area near the intersection of Eubank Blvd. and Eagle Rock.  As mentioned previously, a major channelization/improved conveyance has been proposed for the La Cueva Arroyo from Eubank to Alameda and along the proposed Alameda Corridor.  Concrete box culverts are proposed for the road crossings where the conveyance crosses the following streets.

-	Alameda		Sta 52+53		2-10'x8' CBC

-	Holbrook		Sta 46+35		2-10'x8' CBC

-	Oakland		Sta 56+11		2-10'x8' CBC

- PRIORITIZATION OF INFRASTRUCTURE REQUIREMENTS

- General

The total estimated construction cost for the facilities listed in Table 3, including the proposed La Cueva Channel from Eubank Blvd. to Ventura along the proposed Alameda Corridor but not counting the considerable amount of road related work, is approximately $7,688,390.  This  is a significant amount that is probably beyond the capacity of Bernalillo County to construct in the near future.  Therefore, a system of prioritization was established in order to determine the most critical components.  The method of prioritization is discussed below.

- Scoring System

The scoring system adopted was weighted toward giving threats to life and property the highest priority.  Because avulsions carry increased risks simply because of the uncertainty associated with them, as well as the potential threat to life and property, they were also given extra weight.  The following point system was used:

| TABLE 4 WEIGHTED POINT SYSTEM |  |
| --- | --- |
| CONDITION | POINTS |
| Avulsion, Q100 > 100 cfs | 20 |
| Avulsion, Q100 < 100 cfs | 10 |
| Residential Flooding | 10/dwelling unit |
| Depth of Flow over Road >.67' | 10 |
| Velocity  Depth > 6.5 | 10 |
| Q100 > 200 cfs | 10 |
| 100 cfs < Q100 < 200 cfs | 5 |
| Culvert Hw/d > 1.5 | 5 |

- Results

A total of seventy-two projects have been identified in the Phase II portion of North Albuquerque Acres.  The results of the weighted scoring are shown on Table 5.  The total score for each project ranged from 0 for many of the minor culverts with flow rates less than 100 cfs to 50 for a structure associated with the proposed channelization of the La Cueva Arroyo.  Of the seventy-two projects, thirty-three had a score of 10 or higher.  These are the locations where there is a threat, under certain conditions, to public safety and/or private property.  These projects should be built as soon as funds are available.  Cooperation with transportation agencies and AMAFCA is essential not only in terms of pursuing common strategies but in finding ways to share in the cost and implementation of this plan.

The remaining structures are necessary to fully meet the criteria discussed in this report but do not pose an immediate hazard to the public.  In most cases the installation of these structures can wait until future road and/or utility work is being done in the general area of the proposed structure.

| TABLE 5 -- NORTH ALBUQUERQUE ACRES PROPOSED DRAINAGE STRUCTURES RANKED BY PRIORITY PHASE II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank | Roadway | Zone Atlas Map | Station | Structure | 100-yr Flow Rate cfs (Future) | Avulsion < 100 cfs x 10 | Avulsion > 100 cfs x 20 | Res. Flood x 10 | 100 < Q < 200 x 5 | Q >200 cfs x 10 | Vxd > 6.5 x 10 | Road d > .67 x 10 | Hw/d > 1.5 x 5 | TOTAL |
| 1 | ALAMEDA | C-21 | 52+53 | 2 - 10' x 8' CBC | 2770 |  | 20 |  |  | 10 | 10 | 10 |  | 50 |
| 2 | GLENDALE | B-21 | 25+30 | 4- 54" CMP w/ EMBANKMENT | 685 | 10 |  |  |  | 10 | 10 | 10 |  | 40 |
| 3 | ALAMEDA | C-21 | LA CUEVA CHANNEL | CHANNEL | 2770 |  |  |  |  | 10 | 10 | 10 |  | 30 |
| 4 | ALAMEDA | C-20 | EUBANK- HOLBROOK | CHANNEL | 2796 |  |  |  |  | 10 | 10 | 10 |  | 30 |
| 5 | BEVERLY HILLS | B-19 | 10+10 to 21+30 | 36" STORM DRAIN | 98 | 10 |  | 20 |  |  |  |  |  | 30 |
| 6 | BEVERLY HILLS | B-19 | 69+75 | 5 - 36" CMP | 237 |  | 20 |  |  | 10 |  |  |  | 30 |
| 7 | ELENA DR. | B-19 | 23+50 to 32+00 LT | TRAINING DIKE | 1822 |  | 20 |  |  | 10 |  |  |  | 30 |
| 8 | HOLBROOK | C-21 | 13+70 to 39+00 | 36"- 72" SD to VENTURA | 685 |  |  |  |  | 10 | 10 | 10 |  | 30 |
| 9 | HOLBROOK | C-21 | 68+80 | 2 - 10' X 4' CBC | 652 |  |  |  |  | 10 | 10 | 10 |  | 30 |
| 10 | OAKLAND | C-21 | 56+20 | 2 - 10' X 8' CBC | 2770 |  |  |  |  | 10 | 10 | 10 |  | 30 |
| 11 | SAN DIEGO | B-19 | 56+25 | 5 - 36" CMP | 224 |  | 20 |  |  | 10 |  |  |  | 30 |
| 12 | VENICE | B-19 | 25+00 | 3-48" CMP w/ BERM | 266 |  | 20 |  |  | 10 |  |  |  | 30 |
| 13 | VENICE | B-19 | 44+00 | 3 - 48" CMP | 248 |  | 20 |  |  | 10 |  |  |  | 30 |
| 14 | ALAMEDA | C-20 | 15+30 | 1 - 36" CMP | 43 | 10 |  |  | 5 |  |  | 10 |  | 25 |
| 15 | BEVERLY HILLS | B-19 | 47+30 | 3 - 36" CMP | 119 |  | 20 |  | 5 |  |  |  |  | 25 |
| 16 | BEVERLY HILLS | B-19 | 49+50 | 3 - 36" CMP | 119 |  | 20 |  | 5 |  |  |  |  | 25 |
| 17 | FLORENCE | B-20 | 12+00 to 25+75 | 2 - PAVED SWALES | 124 & 69 |  |  |  | 5 |  | 10 | 10 |  | 25 |
| 18 | BARSTOW | B-19 | 21+10 | 3 - 48" CMP | 245 |  |  |  |  | 10 |  | 10 |  | 20 |
| 19 | HOLBROOK | C-21 | 46+35 | 2 - 10' X 8' CBC | 2773 |  |  |  |  | 10 |  | 10 |  | 20 |
| 20 | WYOMING | B-19 | 24+40 | 3 - 48" CMP | 253 |  |  |  |  | 10 |  | 10 |  | 20 |
| 21 | WYOMING | B-19 | 27+10 | 4 - 48" CMP | 353 |  |  |  |  | 10 |  | 10 |  | 20 |
| 22 | VENTURA | B-20 | 16+00 | 4 - 36" CMP | 193 |  |  |  | 5 |  |  | 10 |  | 15 |
| 23 | VENTURA | B-20 | 24+00 | 2 - 48" CMP | 182 |  |  |  | 5 |  |  | 10 |  | 15 |
| 24 | BARSTOW | B-19 | 11+80 | 3 - 48" CMP | 228 |  |  |  |  | 10 |  |  |  | 10 |
| 25 | BARSTOW | B-19 | 16+30 | 1 - 24" CMP | 19 |  |  |  |  |  |  | 10 |  | 10 |
| 26 | CORONA AVE. | C-21 | 28+50 | 1 - 36" CMP | 79 | 10 |  |  |  |  |  |  |  | 10 |
| 27 | ELENA DR. | B-19 | 26+70 | 10 - 36" CMP | 48 | 10 |  |  |  |  |  |  |  | 10 |
| 28 | ELENA DR. | C-22 | 94+40 | 1 - 24" CMP | 11 | 10 |  |  |  |  |  |  |  | 10 |
| 29 | GLENDALE | B-21 | 36+80 | 1 - 24" CMP | 13 | 10 |  |  |  |  |  |  |  | 10 |
| 30 | GLENDALE | B-21 | 3 +50 to 54+50 | 2 - ARMORED EMBANKMENT | 646/2 |  |  |  |  | 10 |  |  |  | 10 |
| 31 | GLENDALE | B-21 | 41+70 | 4 - 48" CMP | 646/2 |  |  |  |  | 10 |  |  |  | 10 |
| 32 | GLENDALE | B-21 | 50+50 | 3 - 48" CMP | 646/2 |  |  |  |  | 10 |  |  |  | 10 |
| 33 | HOLBROOK | B-21 | 74+70 | 2 - 36" CMP | 95 |  |  |  |  |  |  | 10 |  | 10 |
| 34 | MODESTO | B-20 | 13+50 | 4 - 42" CMP w/LINED CHNL. | 253 |  |  |  |  | 10 |  |  |  | 10 |
| 35 | SAN DIEGO | B-20 | 83+50 | 1 - 24" CMP | 22 | 10 |  |  |  |  |  |  |  | 10 |
| 36 | SIGNAL | C-21 | 50+60 | 1 - 30" CMP | 24 | 10 |  |  |  |  |  |  |  | 10 |
| 37 | ALAMEDA CORR. | C-20 | PASEO DEL NORTE TO VENTURA | STORM DRAINS | VARIES |  |  |  | 5 |  |  |  |  | 5 |
| 38 | SAN DIEGO | B-19 | 22+00 TO 24+30 | PAVED SWALE | 104 |  |  |  | 5 |  |  |  |  | 5 |
| 39 | BARSTOW | B-19 | 26+50 | 3 - 30" CMP | 90 |  |  |  |  |  |  |  |  | 0 |
| 40 | BEVERLY HILLS | B-19 | 63+30 | 1 - 24" CMP | 11 |  |  |  |  |  |  |  |  | 0 |
| 41 | CARMEL AVE. | C-21 | 32+75 | DIP-SECTION | 10 |  |  |  |  |  |  |  |  | 0 |
| 42 | EAGLE ROCK | C-20 | 28+73 | DIP-SECTION | 49 |  |  |  |  |  |  |  |  | 0 |
| 43 | EAGLE ROCK | C-21 | 42+65 | DIP-SECTION | 6 |  |  |  |  |  |  |  |  | 0 |
| 44 | EAGLE ROCK | C-21 | 58+19 | DIP-SECTION | 4 |  |  |  |  |  |  |  |  | 0 |
| 45 | ELENA DR. | B-19 | 56+44 | DIP-SECTION | 17 |  |  |  |  |  |  |  |  | 0 |
| 46 | ELENA DR. | B-19 | 67+00 | DIP-SECTION | 25 |  |  |  |  |  |  |  |  | 0 |
| 47 | FLORENCE | B-20 | 10+35 | 1 - 24" CMP | 8 |  |  |  |  |  |  |  |  | 0 |
| 48 | FLORENCE | B-20 | 12+00 | 2 - 30" CMP | 69 |  |  |  |  |  |  |  |  | 0 |
| 49 | FLORENCE | B-21 | 51+50 | 1 - 24" CMP | 5 |  |  |  |  |  |  |  |  | 0 |
| 50 | HOLBROOK | C-21 | 42+60 to 50+00 | 24" to 48" STORM DRAIN | 95 |  |  |  |  |  |  |  |  | 0 |
| 51 | HOLBROOK | C-21 | 55+40 | 1 - 30" CMP | 23 |  |  |  |  |  |  |  |  | 0 |
| 52 | HOLBROOK | C-21 | 59+50 | 1 - 24" CMP | 5 |  |  |  |  |  |  |  |  | 0 |
| 53 | HOLBROOK | C-21 | 62+20 | 2 - 36" CMP | 78 |  |  |  |  |  |  |  |  | 0 |
| 54 | HOLBROOK | B-21 | 63+90 | 1 - 36" CMP | 45 |  |  |  |  |  |  |  |  | 0 |
| 55 | HOLBROOK | C-21 | 73+20 | 1 - 24" CMP | 8 |  |  |  |  |  |  |  |  | 0 |
| 56 | HOLBROOK | B-21 | 80+00 | 1 - 24" CMP | 11 |  |  |  |  |  |  |  |  | 0 |
| 57 | MODESTO | B-20 | 30+50 | 2 - 30" CMP | 58 |  |  |  |  |  |  |  |  | 0 |
| 58 | MODESTO | B-21 | 41+62 | DIP-SECTION | 13 |  |  |  |  |  |  |  |  | 0 |
| 59 | OAKLAND | C-20 | 19+69 | DIP-SECTION | 17 |  |  |  |  |  |  |  |  | 0 |
| 60 | OAKLAND | C-21 | 39+00 | 1 - 24" CMP | 10 |  |  |  |  |  |  |  |  | 0 |
| 61 | OAKLAND | C-21 | 48+50 | 1 - 24" CMP | 12 |  |  |  |  |  |  |  |  | 0 |
| 62 | OAKLAND | C-21 | 51+50 | 1 - 24" CMP | 14 |  |  |  |  |  |  |  |  | 0 |
| 63 | SAN DIEGO | B-20 | 102+09 | DIP-SECTION | 16 |  |  |  |  |  |  |  |  | 0 |
| 64 | SAN DIEGO | B-20 | 106+24 | DIP-SECTION | 10 |  |  |  |  |  |  |  |  | 0 |
| 65 | SAN DIEGO | B-19 | 38+50 | 1- 24" CMP | 11 |  |  |  |  |  |  |  |  | 0 |
| 66 | SAN DIEGO | B-20 | 63+50 | 1 - 24" CMP | 6 |  |  |  |  |  |  |  |  | 0 |
| 67 | VENTURA | B-20 | 18+30 | 1 - 24" CMP | 11 |  |  |  |  |  |  |  |  | 0 |
| 68 | VENTURA | B-20 | 27+95 | 1 - 24" CMP | 4 |  |  |  |  |  |  |  |  | 0 |
| 69 | WILSHIRE | C-21 | 31+75 | DIP-SECTION | 22 |  |  |  |  |  |  |  |  | 0 |
| 70 | WILSHIRE | C-21 | 37+65 | DIP-SECTION | 4 |  |  |  |  |  |  |  |  | 0 |
| 71 | WYOMING | B-19 | 15+70 | 1 - 30" CMP | 21 |  |  |  |  |  |  |  |  | 0 |
| 72 | WYOMING | B-19 | 18+40 | 1 - 30" CMP | 30 |  |  |  |  |  |  |  |  | 0 |

    - REFERENCES

AVID, 1997, AAlameda East Corridor Location/Environmental Study,@  prepared by Avid Engineering, Inc. for the Bernalillo County Public Works Division, June 1997.

Brater, E.F. and H.W. King, 1982, AHandbook of Hydraulics for the Solution of Hydraulic Engineering Problems.@  Sixth Edition, McGraw-Hill, Inc.

COA, 1997, ADevelopment Process Manual, Volume 2 -- Design Criteria,@ Chapter 22, Drainage, Flood Control and Erosion Control, prepared by the DPM Drainage Committee.

FEMA, 1996, Restudy for North Albuquerque Acres for the North Domingo Baca, South Domingo Baca and Pino Arroyos, Community No. 35002.  Prepared by Resource Technology, Inc., Contract No. EMW-94-C-4647, Submitted May 31, 1996.  Draft maps have been prepared at the time of this writing.

FHWA, 1985, AHydraulic Design of Highway Culverts,@ Hydraulic Design Series No. 5 (HDS-5), U.S. Dept of Transportation.

RCE,1994, ASediment and Erosion Design Guide,@ prepared by Resource Consultants and Engineers, Inc. for AMAFCA. November, 1994.

RTI,1991, ANorth and South Domingo Baca and Paseo del Norte Corridor Drainage Master Plan,@prepared by Resource Technology, Inc. for the Albuquerque Metropolitan Arroyo Flood Control Authority (AMAFCA).  December 1991.

RTI, 1998a, AHydrology Report for the North Albuquerque Acres/Sandia Heights Master Drainage Plan,@  prepared by Resource Technology, Inc. for the Bernalillo County Public Works Division, November 1998.

RTI, 1998b, ANorth Albuquerque Acres Master Drainage Plan B Final,@ Prepared by Resource Technology, Inc. for the City of Albuquerque Public Works Department, October 1998.

RTI 1999a, ANorth Albuquerque Acres and Sandia Heights South Drainage Study Infrastructure Plan - Phase I,@ prepared by Resource Technology, Inc. for Bernalillo County Public Works Division, June 1999.

RTI,1999b, ALa Cueva, El Camino and North Camino Master Drainage Plan,@ prepared by Resource Technology, Inc. for AMAFCA. July, 1999.

RTI, 1999c, ANorth Albuquerque Acres Site Grading and Drainage Report,@  prepared by Resource Technology, Inc. for the Bernalillo County Public Works Division, June 1999.

T. Mann and Associates, 1994.  Aerial photogrammetric mapping and topographic survey of a portion of the North Albuquerque Acres/Sandia Heights area for the Albuquerque Metropolitan Arroyo Flood Control Authority.  Thomas R. Mann and Associates, Albuquerque, New Mexico.  Flown June 7, 1994.