October 31, 2013

Philip W. Clark, P.E.

Clark Consulting Engineers

19 Ryan Road

Edgewood, NM 87015

Re:	Gillani Residence, Grading and Drainage Plan

Lot 10, Block 1, Tract 3, Unit 3, NAA

8800 Modesto Ave., NE

Engineer’s Stamp dated 9-30-13 (C20/D068)

Dear Mr. Clark,

Based upon the information provided in your submittal received 10-03-13, the above referenced Grading and Drainage Plan cannot be approved for building permit based on the following comments:

- The contractors in most cases grade/disturb the entire lot. Therefore, under developed conditions land treatment A should be zero.  Please also double check land Treatment D.

- Provide an offsite basin map.  How much runoff will be draining through the 18” CMP proposed in the road?

- Provide culvert capacity calculations for all the pipes.  Is the swale north side of the property sized adequately to carry the offsite runoff from the east?

- Provide detail elevations for the entrances and the culvert crossing.  Please include edge of the asphalt along Modesto and top of culvert elevations.

- Provide riprap at the outlet of the pipes.

Recommendation:

- Carry the offsite runoff from southeasterly portion of the lot to the north to Modesto in a swale and avoid building additional culvert and improvement in front of the Garage.

If you have any questions, you can contact me at 924-3695.

Sincerely,

Shahab Biazar, P.E.

Senior Engineer, Planning Dept.

Development Review Services

C:	e-mail