July 31, 2026

Richard J. Berry, Mayor

Phillip W. Clark, PE

Clark Consulting Engineers

19 Ryan Rd

Edgewood, NM 87015

RE:	Signal Points lots 19 & 20

Block 4, Tract 3, Unit 3, N. Albuq. Acres

Grading and Drainage Plan

Engineer’s Stamp Date:  3/2/15 (C20D070)

Dear Mr. Clark:

Based upon the information provided in your submittal received 3/2/15, the above referenced plan cannot be approved for Preliminary Plat by the DRB, until the following comments are addressed,

- Provide the .34 times the impervious area for each home and the entire subdivision for first flush requirement.

- The arroyo breached the Flood Plain limits and passes thru his site as shown by the aerials and site visits.  There have been numerous conversations on acceptable mechanisms to deal with this scenario.  These means were delineated in letter dated 3-21-14 and are in the drainage file.  Upon further conjecture, item 2a. will require a LOMR.

- Quantify flows from the La Cueva Arroyo breach and provide HES-RAS calculations.

- Provide adequate protection from the arroyo.

The pond shown will be fine for the management of the first flush.

If you have any questions, you can contact me at 924-3695.

Sincerely,

Rita Harmon, P.E.

Senior Engineer, Planning Dept.

Development Review Services

Orig:	Drainage file

c.pdf:	via Email:  Recipient, Monica Ortiz