June 11, 2015

- Scour Depth – use base of the arroyo

- Flow amount to be diverted to the north and capacity calculations

- Floodwall changes and details.  The wall should be extended along the westerly property line.

- Easement should be maintained form the adjacent property owners for the proposed improvements within their site.  Who will maintain these improvements?  Construction easements will be required from adjacent property owners to construct the floodwall.

- Development runoff from Subdivision to be diverted to the north (maintain drainage easement on one lot) and obtain approval form AMFACA.  Otherwise drainage easement will be required.  Provide erosion protection and the point of discharge.

- Show first flush pond on each lot.  First Flush Pond can be on Lot 6 if the developed runoff is being routed to the west.  The difference in proposed developed flows and allowable flows can be retained in the same pond.

- The proposed runoff exceeds the allowable runoff based on the NAA DMP proposed land treatments.