|  | Date: 02/15/19 |  |  |  |
| --- | --- | --- | --- | --- |
| Project: | Paaru Builders for Holly NIDO center. |  |  |  |
| # Corrective Action Log |  |  |  |  |
| Number | Corrective Action Needed | Date Action Noted | Date Corrected | Date Correction Noted By Inspector |
| 01 | BMP's in place prior to earth disturbing activities. Missing sections of silt fence, aprox. 500', required in the West section per the Erosion Control Plan. | 08/16/18 | Inspections Plus to install per PM direction. | 08/23/18 |
| 02 | Track out pad required at West exit, near stockpiles, where sediment is being transferred to Holly Ave. | 08/16/18 | Exit to be closed off per PM. | 08/30/18 |
| 03 | Post NOI and provide SWPPP book on site for inspector review. | 08/16/18 |  | 08/30/18 |
| 04 | No additional site BMP corrective actions noted at the time of inspection. | 08/23/18 |  | 08/23/18 |
| 05 | Additional rock to be added to trackout pad, east side. | 08/30/18 | Inspections Plus to maintain 09/12/18 | 09/13/18 |
| 06 | Silt fence to be extended at edge of trackout pad, east side. | 08/30/18 | Inspections Plus to maintain 09/12/18 | 09/13/18 |
| 07 | No additional site BMP corrective actions noted at the time of inspection. | 09/06/18 |  | 09/06/18 |
| 08 | No additional site BMP corrective actions noted at the time of inspection. | 09/13/18 |  | 09/13/18 |
| 09 | No additional site BMP corrective actions noted at the time of inspection. | 09/20/18 |  | 09/20/18 |
| 10 | BMP's in good working condition. No sign of sediment discharge from site. No corrective actions noted. | 09/27/18 |  | 09/27/18 |
| 11 | Install silt fence on the South side of the project, where block wall has been removed. | 10/11/18 |  | 11/08/18 |
| 12 | Rain event report completed on 10/24/18. Rain inspection reflets 1.60" at time of inspection. | 10/24/18 |  | 10/24/18 |
| 13 | No site BMP corrective actions noted at the time of inspection. Block wall in progress South side. | 11/08/18 |  | 11/08/18 |
| 14 | No site BMP corrective actions noted at the time of inspection. Block wall continues South side. Continue to monitor concrete roll off as it's getting full. | 11/22/18 |  | 11/22/18 |
| 15 | Please dispose of the wash-out material and any wetted soil on the ground, next to the concrete roll off. | 12/06/18 |  | 01/03/19 |
| 16 | No additional site BMP corrective actions noted at the time of inspection. | 12/20/18 |  | 12/20/18 |
| 17 | No additional site BMP corrective actions noted at the time of inspection. Storm event on 12/26-27/18 recorded at: 0.24" | 01/03/19 |  | 01/03/19 |
| 18 | Collect trash/ debri in the North East corner of the project and place in the onsite dumpster. | 01/17/19 |  | 02/01/19 |
| 19 | Remove concrete washout piles from the site, place in the onsite concrete roll off system. | 01/17/19 |  | 02/01/19 |
| 20 | Storm Events per Weather Underground: Wyoming and Paseo del Norte (Mark's) KNMALBUQ56 : 01/10/19=0.12"; 01/11/19=0.15"; 01/13/19=0.12" . | 01/17/19 |  | 01/17/19 |
| 21 | No site BMP corrective actions noted at the time of inspection. Utility tie-in work on the East end. Stockpile should be removed and silt fence should be repaired as soon as possible at tie-in location. | 02/01/19 |  | 02/04/19 |
| 22 | No site BMP corrective actions noted at the time of inspection. Final grading in progress. Storm event 02/15/19= 0.12" per Weather Underground: Station 1 KNMALBUQ467 | 02/15/19 |  | 02/15/19 |
| SWPPP Management Signature: Date: |  |  |  |  |