August 18, 2011

Joseph J. Casares, Jr. P.E.

Larkin Group NM Inc.

8500 Menaul Blvd NE, Suite A-440

Albuquerque, NM 87112

Re:	International Indoor Soccer Area, 1311 Cuesta Arriba Ct. NE, Grading Plan Engineer’s Stamp date 7-14-11 (D16/D003)

Dear Mr. Casares,

Based upon the information provided in your submittal received 8-11-11, the above referenced project cannot be approved for building permit until the following comments are addressed:

- Provide a drainage submittal sheet.  Make sure that a SO-19 approval is requested on that.  A $50.00 fee submittal is required.

- Is the address 1311 or 1321 Cuesta Arriba Ct., NE.

- Please provide calculations for the runoff analysis (if AHYMO was used please submit your input and summary output files)

- Provide City Drainage file number and a copy of the plan where the allowable discharged is shown.

- Provide the total discharge from the site and make sure that it does not exceed the allowable discharge.

- Provide SO-19 construction notes and signature block for the construction of the sidewalk culvert.

- Please use the attached revised copy of the sidewalk culvert on your plans.

- Extend the sidewalk culvert plate to the back of the curb.

- Also use the orifice equation for sizing the sidewalk culvert.

- Thicken the boundaries for drainage Section A, B, & C.

- Please provide spot elevations for the trash enclosure.

- Please show parking spaces and handicap parking area.

- Please show the door locations for the building.

Page 2

August 18, 2011

- The landscaping along the westerly property line is draining out over the sidewalk.  Please provide a sidewalk culvert, or route the runoff onto the paving area and out to the street through the entrance.

- See attached markups for additional comments.

- Hydrology is requesting that proposed landscape areas be depressed to retain/infiltrate the rain that falls on them.

If you have any questions, you can contact me at 924-3695.

Sincerely,

Shahab Biazar, P.E.

Senior Engineer, Planning Dept.

Development and Building Services

# C:	file