October 24, 2019

Hunaman Corporation Sam Patel President taurus5468@gmail.com

Site: True by Hilton,. NE, F17E044C

Re:	Drainage Ordinance (14-5-2-1) Violations for Erosion and Sediment Control

Dear Mr. Patel,

The following violations were observed at the inspections on 10/17/2019.

- Stormwater controls are in need of repair (Silt Fence).

- Sediment in the street.

Photos are shown below.

The following violations were observed at the inspections on 10/24/2019.

- Sediment in the street, HC ramp, sidewalk, rundown at south end of street and piled on adjacent private.

Photos are shown below.

- Paint and Concrete wash water.

- Construction and domestic waste not in acceptable container

Required Mitigation:

- Remove and properly dispose of all offsite sediment and prevent sediment from leaving the site in the future.

- Direct wash water from stucco, paint and concrete into a leak-proof container or lined pit. Remove and properly dispose of existing washouts.

- Dispose of construction and domestic waste in acceptable containers and use lids or tarps to cover waste containers when not in use.

If the violations are not mitigated within 7 days, the property owner is subject to a fine of $500/day per the City’s Drainage Control Ordinance.

If you have any questions, you can contact me at 924-3420 or jhughes@cabq.gov

Sincerely,

James D. Hughes, P.E.

Principal Engineer, Hydrology/Stormwater Quality

Planning Dept.