January 11, 2012

Joseph Barela, P.E.

Joe B. Engineering, Inc.

P.O. Box 5734

Santa Fe, NM 87502

Re:	Iden-Samora Compound Grading and Drainage Plan

Engineer’s Stamp Dated 12-15-11 (G13/D028)

Dear Mr.  Barela,

Based upon the information provided in your submittal received 1-3-12, the above referenced plan cannot be approved for Platting action by the DRB until the following comments are addressed:

- Per the DPM, sites that use retention ponding are to retain the developed flows for the 10-day storm.  Since this site is flat, currently does not have a pond, and to accommodate the development of these lots, Hydrology would agree to providing ponding for the 6-hour storm difference, as previously shown, plus the 10-day storm for proposed impervious surfaces.  So, for Lot B this would be 704 cu. ft (difference) plus 507 cu. ft. (impervious 10-day) for a total of 1211 cu ft.  Standard retention ponding would require 2550 cu. ft.

- Provide volume calculations.

- It seems depressing a larger area, less deep, would accomplish the necessary ponding as well as keeping the landscape areas useful.

- It appears that Lot A will drain to Lot B (68.9 to 68.4).  This amount should be noted and accounted for.  It could also be ponded on Lot A.

- Where will the swale be built that is detailed on Sheet GD-2?

- Show the high point/water block.

- Show the proposed driveway locations.  Most homeowners will not want the pond to be in the driveway.

- Provide a couple proposed spot elevations in the landscape areas so it is easier to see the drainage path to the pond(s).

- Provide benchmark information.

- Provide a Vicinity Map.

- Provide the Flood Zone designation.

If you have any questions, you can contact me at 924-3986.

Sincerely,

Curtis Cherne, P.E.

Principal Engineer, Planning Dept.

Development and Building Services

# C:	joebengineering@msn.com