October 18, 2011

David Soule, P.E.

Rio Grande Engineering

P.O. Box 67305

Albuquerque, NM 87193

Re:	HUESTAS DE LA FE Church Expansion

Grading and Drainage Plan

Engineer’s Stamp date 8-10-11 (G-15/D104)

Dear Mr. Soule,

Based upon the information provided in your submittal received 10/07/11, the above referenced plan cannot be approve for Building Permit or Grading Permit until the following comments are addressed:

- Show the AH flood zone at the west end of this site on the plan.

- Is this addition being placed over existing concrete? If so, what does keyed not 4 pertain to?

- It is difficult to confirm your land treatment calculations for this site please adjust your plan accordingly.

- Give elevation points from the surrounding sites.

- Prior to CO, an Elevation Certificate for the existing structure in the AE Flood Zone is required.

- A Floodplain Development Permit is required for the proposed building in the AE Flood Zone.

- Provide routing calculations for the pond to verify maximum discharge and the 100 year WSEL.

- Remove all notes pertaining to the French drain.

- Is this site being repaved or is it being repaired? Are the Islands providing landscaping or are they just painted?

- It appears that the water blocks are in the wrong locations, does this change your calculations?

- At the area drain location, what is the land treatment for this area? It does not appear to provide any slope toward the inlet.

- Call out flowline elevations at the Northern entrance on 2nd St.

Hydrology is requesting that proposed landscape areas be depressed to retain/infiltrate the rain that falls on them.

If you have any questions, you can contact me at 924-3695, or Rudy Rael at 924-3977.

Sincerely,

Shahab Biazar, P.E.

Senior Engineer, Planning Dept.

Development and Building Services

RER/SB

C:	File