Driveway A has poor LOS in all conditions existing and proposed.  The failing movements are the northbound combination through/left-turn LOS F, southbound right/left/through movements LOS F and the westbound left LOS F.  The opposing traffic volume on Norma Dr. (southbound) is very low but results in a LOS of F.  Recommendation is prohibiting the southbound left turn and through movement from Norma Dr and the northbound left turn from driveway A and the westbound left turn from Montgomery into the development.  The median opening on Montgomery across from driveway A should be removed.

Show the access for Taco Cabana on the site plans.  At driveway B relocate the access for Taco Cabana at least 75 feet away from Montgomery Blvd.

Define the acronym F.A.R in the conceptual site plan (0.25 F.A.R.)

Warranted right turn lanes:

- Driveway A

  - Eastbound Montgomery at Driveway A, Montgomery no turn lane in existing condition.

  - Northbound Driveway A, not required if right-in/right-out access

- Driveway B

  - Eastbound Montgomery at Driveway B; the existing right turn lane is shorter than recommended in the DPM.  Combining the transition and deceleration length is allowed and results in approximately 240 feet which can be permitted.

  - Northbound Driveway B, not required it is right-in/right-out access

- Driveway C

  - Eastbound at driveway C on Wyoming, can be done with lane striping.

  - Southbound Wyoming at driveway C

  - Northbound Wyoming at driveway C.   The existing length is acceptable when combining the transition and deceleration lengths.

- Montgomery/ Wyoming intersection

  - Montgomery/ Wyoming intersection all right turn movements warrant turn lanes. The development contributes traffic to the eastbound right-turn movement at Wyoming. The other right turn movement have little or no traffic due to the development.  A eastbound right turn lane should be added at the Montgomery/Wyoming intersection.

- Wyoming/ La Mirada

  - Wyoming/ La Mirada, eastbound re-stripe with a right-turn lane, thru/left lane

Warranted left turn lanes:

- Northbound Wyoming

  - Northbound Wyoming at La Mirada existing shorter than recommended.  Provide expected queue length to assess the existing condition acceptability

- Driveway A

  - Driveway A Northbound left turn

- Montgomery/ Wyoming intersection

  - All left turns have dual left lanes