04/05/19_Follow-up Inspection for H09E017F—Pulte Homes

Some outstanding issues were addressed; however, same issues observed from previous inspections:

- 2312 Gold Butte—mortar/Cementous washout in front yard. Washout around mixer, on southwest portion and at curb line at driveway.  (three photos)

- Wind Caves—on previous inspection this roadway was simplified (explained as) excessive washout.  These photos clarify the lots, by address, for more than minor and unacceptable washout and litter and construction debris.

- 9204 Wind Caves has oil spill in roadway, appears from slab construction—pump truck or cement truck,  this oil spill extends from address north onto Komatke approx. 75lf.  This requires immediate mitigation.

- Several lots have recent, with in the past 4hrs, cement washout.

- Subcontractors not utilizing refuse locations or cleaning/collecting trash (litter).  Soda, water, Monster, and domestic litter is mixed in with construction debris—other than the allowed wood.

- Some lots have sheetrock stored or left outside in front yards.  This requires mitigation.

Previous e—mail sent to Contractor identified to mitigate and train preventing these types of infractions and/or issues.  On this site, the issues seem to be ongoing or cycling.

With the violation follow-up a more extensive inspection is necessary due to additional homes being constructed on lots.