|  | Date: 07/18/19 |  |  |  |
| --- | --- | --- | --- | --- |
| Project: | Albuquerque RV and Boat Storage |  |  |  |
| # Corrective Action Log |  |  |  |  |
| Number | Corrective Action Needed | Date Action Noted | Date Corrected | Date Correction Noted By Inspector |
| 01 | Provide SWPPP board and NOI. | 10/09/17 |  | 10/24/17 |
| 02 | Stabillized Construction pads in progress at time of inspection. | 10/09/17 |  | 10/09/17 |
| 03 | Install Silt fence as required by ESCP. | 10/09/17 |  | 10/24/17 |
| 04 | Provide rain gauge onsite. | 10/24/17 |  | 11/06/17 |
| 05 | Provide SWPPP book onsite. | 10/24/17 |  | 11/06/17 |
| 06 | Silt fence along South side at the entrance, West of Blue bell, is not properly keyed into ground. Requires trenching for silt fence or wattle placement. | 10/24/17 |  | 11/06/17 |
| 07 | No site BMP corrective actions noted at time of inspection. | 11/06/17 |  | 11/06/17 |
| 08 | No site BMP corrective actions noted at time of inspection. | 11/20/17 |  | 11/20/17 |
| 09 | No site BMP corrective actions noted at time of inspection. | 12/04/17 |  | 12/04/17 |
| 10 | No site BMP corrective actions noted at time of inspection. | 12/19/17 |  | 12/19/17 |
| 11 | No site BMP corrective actions noted at time of inspection. | 01/02/18 |  | 01/02/18 |
| 12 | No site BMP corrective actions noted at time of inspection. | 01/16/18 |  | 01/16/18 |
| 13 | No site BMP corrective actions noted at time of inspection. | 01/30/18 |  | 01/30/18 |
| 14 | No site BMP corrective actions noted at time of inspection. Clean site, block wall continues. | 02/12/18 |  | 02/12/18 |
| 15 | No site BMP corrective actions noted at time of inspection. | 02/27/18 |  | 02/27/18 |
| 16 | No site BMP corrective actions noted at time of inspection. | 03/13/18 |  | 03/13/18 |
| 17 | No site BMP corrective actions noted at time of inspection. Block wall continues along West side. | 03/26/18 |  | 03/26/18 |
| 18 | Culvert protection required at South end of project, place wattle across face of sidewalk culvert. There is a tiered mulch sock system but sediment is escaping through culvert to La Morada. Picture attached to e mail. | 04/10/18 |  | 04/24/18 |
| 19 | Repair/ replace approx. 70' of silt fence on the North end staging area just East of concrete washout. | 04/10/18 |  | 04/24/18 |
| 20 | Curb sweep required on La Morada, in particular at the construction entrance. Spring winds will continue to cause problems. | 04/10/18 |  | 04/24/18 |
| 21 | No site BMP corrective actions noted at time of inspection. Block wall continues along West side. | 04/24/18 |  | 04/24/18 |
| 22 | Remove built up sediment on mulch socks at outfall, SE corner near La Morada. | 05/08/18 | 05/13/18 | 06/05/18 |
| 23 | Repair outfall in SE corner. Sediment bypassed mulch socks, dirt on La Morada. Street sweeping required. | 05/22/18 | 05/23/18 | 06/05/18 |
| 24 | No site BMP corrective actions noted at time of inspection. | 06/05/18 |  | 06/05/18 |
| 25 | No site BMP corrective actions noted at time of inspection. | 06/13/18 |  | 06/13/18 |
| 26 | No site BMP corrective actions noted at time of inspection. | 06/19/18 |  | 06/19/18 |
| 27 | No site BMP corrective actions noted at time of inspection. | 06/26/18 |  | 06/26/18 |
| 28 | No site BMP corrective actions noted at time of inspection. | 07/03/18 |  | 07/03/18 |
| 29 | No site BMP corrective actions noted at the time of inspection. Clean site. Block wall continues South to East at property boundry. | 07/17/18 |  | 07/17/18 |
| 30 | Repair silt fence/ wattle in SE corner near outfall on La Morada. | 07/31/18 | Cut back in place. | 01/16/19 |
| 31 | No additional site BMP corrective actions noted at time of inspection. | 08/14/18 |  | 08/14/18 |
| 32 | No additional site BMP corrective actions noted at time of inspection. | 08/28/18 |  | 08/28/18 |
| 33 | No additional site BMP corrective actions noted at time of inspection. | 09/11/18 |  | 09/11/18 |
| 34 | No additional site BMP corrective actions noted at time of inspection. | 09/25/18 |  | 09/25/18 |
| 35 | No additional site BMP corrective actions noted at time of inspection. | 10/09/18 |  | 10/09/18 |
| 36 | Stormwater escaping from NE corner outfall to inlet on Vista Oriente and La Morada. Repair wattles or provide inlet protection. Rain event recorded: 1.10" on 10/24/18. | 10/24/18 |  | 11/06/18 |
| 37 | No additional site BMP corrective actions noted at time of inspection. Rain event on 10/31/18 recorded at 0.22" on nearby rain gauge at 9 am. | 11/06/18 |  | 11/06/18 |
| 38 | No site BMP corrective actions noted at the time of inspection. Tie-in work at Southwest corner, streets and curbs clear of sediment transfer. | 11/20/18 |  | 11/20/18 |
| 39 | No site BMP corrective actions noted at the time of inspection. Sidewalks and drivepads in progress South end. NOI reposted on construction board. | 12/06/18 |  | 12/06/18 |
| 40 | No site BMP corrective actions noted at the time of inspection. Outfall at La Morada cleared and protected with appropriate BMP's. | 12/20/18 |  | 12/20/18 |
| 41 | No site BMP corrective actions noted at the time of inspection. Storm event on 12/26-27/18 recorded at : 0.24". | 01/03/19 |  | 01/03/19 |
| 42 | No site BMP corrective actions noted at the time of inspection. Storm events: 1/10/19= 0.13"; 1/11/19= 0.05"; 1/13/19= 0.11" | 01/16/19 |  | 01/16/19 |
| 43 | No site BMP corrective actions noted at the time of inspection. Storm events: 1/18/19= 0.09". Sidewalks complete South end. Utility tie in work in progress South end. | 01/31/19 |  | 01/31/19 |
| 44 | No site BMP corrective actions noted at the time of inspection. Storm events: 2/14/19= 0.12". | 02/14/19 |  | 02/14/19 |
| 45 | No site BMP corrective actions noted at the time of inspection. Storm events: 2/19/19= 0.12"; 02/22/19= 0.03"; 02/23/19= 0.16" per Weather Underground: Mirehaven - Del Webb KNMALBUQ509 | 02/28/19 |  | 02/28/19 |
| 46 | No site BMP corrective actions noted at the time of inspection. Storm events: 3/11/19= 0.22"; 03/12/19= 0.42" per Weather Underground: Mirehaven - Del Webb KNMALBUQ509 | 03/13/19 |  | 03/13/19 |
| 47 | No site BMP corrective actions noted at the time of inspection. | 03/28/19 |  | 03/28/19 |
| 48 | No site BMP corrective actions noted at the time of inspection. High winds, BMP's in good working order. Storm event on 03/31/19= 0.10" | 04/10/19 |  | 04/10/19 |
| 49 | Storm event on 04/17/19= 0.50" per Weather Underground: Mirehaven - Del Webb KNMALBUQ509. Storm inspection report completed on 04/18/19. | 04/18/19 |  | 04/18/19 |
| 50 | No site BMP corrective actions noted at the time of inspection. | 04/25/19 |  | 04/25/19 |
| 51 | Install wattles in the SE corner at outfall and unused drivepad location. | 05/09/19 | Cut back maintained. | 05/24/19 |
| 52 | Shovel sediment away from wattle and reposition wattles to cover the North entrance. | 05/09/19 |  | 05/24/19 |
| 53 | Storm event on 05/10-11/19= 0.30" per Weather Underground: Mirehaven - Del Webb KNMALBUQ509. Storm inspection report completed on 05/13/19. | 05/13/19 |  | 05/13/19 |
| 54 | No site BMP corrective actions noted at the time of inspection. Storm event on 05/13/19= 0.05". Leonard on site for contact. | 05/23/19 |  | 05/23/19 |
| 55 | No site BMP corrective actions noted at the time of inspection. Active watering on the East end. Xeriscape/ landscape in progress South side. | 06/06/19 |  | 06/06/19 |
| 56 | Wattles required at the SE outall on La Morada and the SE construction exit, usually closed, behind the cut back. | 06/20/19 |  | 07/05/19 |
| 57 | Clear built up sediment from the BMP wattles protecting the North construction exit. | 06/20/19 |  | 07/05/19 |
| 58 | No additional site BMP corrective actions noted at time of inspection. | 07/05/19 |  | 07/05/19 |
| 59 | Shovel sediment away from wattle and reposition wattles to cover the North entrance. | 07/18/19 |  |  |
| 60 | Remove domestic debris from SE pond. Storm event on 07/16/19= 0.10" | 07/18/19 |  | 07/18/19 |
| SWPPP Management Signature: Date: |  |  |  |  |