December 1, 2011

James D. Hughes, P.E.

Mark Goodwin & Associates

PO Box 90606

Albuquerque, NM 87199

Re:	Sawmill Crossing, Conceptual Grading Plan

Engineer’s Stamp dated 11-10-11 (H13/D057)

Dear Mr. Hughes,

Based upon the information provided in your submittal received 11-10-11, the above referenced plan cannot be approved for Site Plan for Subdivision action by the DRB until the following comments that were presented at the 11-16-11 DRB meeting are addressed:

- Meeting the ponding requirement will be more difficult with the proposed zoning.  Hydrology would prefer to keep the existing zoning.

- Residential zoning may be possible with a neighborhood pond.

- Proposing public streets with this design will cause a mixture of Public and Private stormwater.  That is undesirable.

- The Conceptual Grading Plan states to use 70% impervious, whereas the DPM has a max of 45% for the flat grading scheme.  The proposed development is too dense to accommodate the ponding required.

- Homeowners are regularly filling in ponds on their lots. Hydrology would prefer to see a more centralized neighborhood private pond.  The pond could double as a park.

- Why does the plan propose to fill so much at the south west corner?  This is a historic outfall location.

If you have any questions, you can contact me at 924-3986.

Sincerely,

Curtis Cherne, P.E.

Principal Engineer, Planning Department

Development and Building Services

Copy:	e-mail