January 20, 2012

Craig W. Hoover, P.E.

Bohannan Huston, Inc.

7500 Jefferson St NE

Albuquerque, NM 87109

Re:	Ladera Dam 5 and Dam 9 Arroyos: APS Projects DMP

Engineer’s Stamp dated-no stamp- December 2011 (J08/D000)

Dear Mr. Hoover,

Based upon the information provided in your submittal received 12-27-11, the above referenced report DMP cannot be approved for Grading Permit until the following comments are addressed:

- Provide plan-size exhibits that show basins, flows, proposed and existing infrastructure, platted and future streets, storm drain capacity and storm drain flows.  I would refer you to BHI UNM Commercial District or MDS.

- How long does it take Pond 5S to drain?  Is the 24-hour storm appropriate?

- Please explain what is expected for water quality inlets.

- The West I40 DMP shows pond 5S as 49 ac-ft, whereas this report shows it as 30 ac-ft.

- Provide a short narrative for the Stormcloud subdivision including the capacity of the storm drain that the 36” proposed storm drain ties into.

- Provide the WSE on the exhibit showing pond 5S (currently Figure 3).

- Is Pond 5S required to protect the APS stadium?  If so, should it be built before the building?

- Show the flood plains.

- Show Arroyo Vista Blvd in plan and profile where the 54” storm drain crosses it.

- Provide calculations for the ported riser in pond 5S.

- A Grading Plan is required to obtain a Grading Permit.

- Hydrology is storing files electronically now.  It would be helpful to put the AHYMO output on a CD in PDF format.

- A CLOMR/LOMR will be required.

Sincerely,

Curtis A. Cherne, P.E.

Principal Engineer, Planning Dept.

Development and Building Services

# C:	choover@bhinc.com