|  | Date: 06/06/19 |  |  |  |
| --- | --- | --- | --- | --- |
| Project: | Hart for Sawmill Marketplace Ph 1. |  |  |  |
| # Corrective Action Log |  |  |  |  |
| Number | Corrective Action Needed | Date Action Noted | Date Corrected | Date Correction Noted By Inspector |
| 01 | Provide SWPPP book on site for inspector review. | 01/03/19 | 01/07/19 | 01/16/19 |
| 02 | Post NOI and rain gauge near construction entrance. | 01/03/19 | Onsite at time of inspection. | 01/03/19 |
| 03 | Owner and Operator of the project need to sign the certification page in the SWPPP book, see section 2. | 01/16/19 |  |  |
| 04 | Operator of the project need to sign the delegation of authority in the SWPPP book, see section 2. | 01/16/19 |  |  |
| 05 | No site BMP corrective actions noted at the time of inspection. Storm event on 01/18/19 recorded at: 0.09" | 01/31/19 |  | 01/31/19 |
| 06 | Additional fractured rock required on construction exit, trackout pad. Slight sediment transfer to roadway. | 02/14/19 |  | 02/15/19 |
| 07 | Replace weighted wattles across construction exit to the West, currently blocked off for tie-in work. | 02/14/19 |  | 02/15/19 |
| 08 | No site BMP corrective actions noted at the time of inspection. Mr. Anderson not on site at the time of inspection. Storm events per Weather Underground: 02/19/19= 0.02"; 02/22/19=0.08"; 02/23/19=0.07" | 02/28/19 |  | 02/28/19 |
| 09 | No site BMP corrective actions noted at the time of inspection. No sign of offsite sediment trackout or concrete washout piles on site. Storm events per Weather Underground and on site gauge. 03/11/19= 0.18"; 03/12/19=0.38" | 03/13/19 |  | 03/13/19 |
| 10 | Remove concrete washout/ soiled ground at the roll off location. | 03/28/19 |  | 04/10/19 |
| 11 | No site BMP corrective actions noted at the time of inspection. No sign of offsite sediment trackout. Storm event per Weather Underground and on site gauge. 03/31/19= 0.10". | 04/10/19 |  | 04/10/19 |
| 12 | Storm event per Weather Underground and on site gauge. 04/17/19= 0.50".Storm inspection completed. | 04/18/19 |  | 04/18/19 |
| 13 | No site BMP corrective actions noted at the time of inspection. | 04/25/19 |  | 04/25/19 |
| 14 | No site BMP corrective actions noted at the time of inspection. Storm event per on site gauge. 04/22/19= 0.10". | 05/09/19 |  | 05/09/19 |
| 15 | Storm events: 05/10/19=0.20"; 05/11/19=0.07" Separate storm events on 05/10-11/19. Storm Event: 05/13/19=0.05" | 05/13/19 |  | 05/13/19 |
| 16 | No site BMP corrective actions noted at the time of inspection. Silt fence repaired from wind storms. No sign of sediment track out from the project site. | 05/24/19 |  | 05/24/19 |
| 17 | Construction and domestic waste not in acceptable container. Please pick up and retain all plastic water bottles and construction material boxes (etc.) Re: J13E017A Sawmill Market Inspection. | 06/06/19 |  |  |
| 18 | Washout on south side of building. Ensure concrete washout facility is utilized for these occasions. Re: J13E017A Sawmill Market Inspection | 06/06/19 |  |  |
| 19 | Fabric fence at 19th and Bellemah to be re-attached to security fence. | 06/06/19 |  |  |
| SWPPP Management Signature: Date: |  |  |  |  |