June 2, 2024	DRAFT DRAFT DRAFT

Luay Aboona

KOLA, Inc.

9575 West Higgins Road, Suite 400

Rosemont, Il 60018

Re:	Proposed Dunkin Drive-Through,

9700 Central Ave. SW

Traffic Impact Study draft, HT#K09D054

Report dated May 22, 2024

Engineer’s Stamp ------

Via email laboona@koainc.com

Dear Mr. Aboona,

The subject Traffic Impact Study Draft (Study) received 5-22-24, has been reviewed by the City of Albuquerque Planning Development Transportation Section.  The City has the following comments to be addressed.

- Adjust the Study and analysis based upon the following general concepts:

  - Relocating the right-in driveway on 98th St to approx. 310 ft south of Central Ave

  - One driveway on Central Ave as shown in the May 2022 Study.

  - Removing or minimizing conflicts at Central and Westland Rd.

- The Study assumes all traffic leaving the site will drive east to Unser Blvd to travel east on I-40.  It should be assumed that a fraction (25%) of the traffic will perform a U-turn at Westland Road where there are conflicts with existing movements and the possibility of traffic backing-up onto Central Ave. This should be discussed in more detail in the Study.

  - Analyze the capacity of the left turn bays on Central Ave turning north onto Unser Blvd.

  - Analyze the feasibility of a median U-turn (Michigan Left) between Westland Rd and 94th St.

- Analyze the capacity of the left turn bays on Central Ave turning North onto Unser Blvd.

- The design of the two right-in/right-out access drives on Central Ave presents a conflict wherein, traffic driving east on Central Ave. may enter the turn bay but not turn into the west access drive, but continue to the east access drive and traffic leaving the site via the west access drive may assume they are turning into the west access drive.

- One access drive as shown on the Study dated May 2022 is preferred.

- Show the street improvements form the West Central Complete Streets_Concept Layount_090-11-20_reduced.pdf plan. The plan is attached to the email.

- The geometry of the right-in access on 98th st may change to shorten the travel distance for pedestrians.

Previous comments that were not addressed:

- A proposed driveway on 98th St. is located too near the intersection of Central Ave. and 98th St. The City’s development Process Manual (DPM) in Table 7.4.45 Minimum Distance Between Commercial Site Access and Intersection requires the access distance to be a minimum of 300 feet from the approach of the intersection.  The existing driveway on 98th is approximately 125’ south of Central Ave.  Correction is needed in the report on page 11 where it claims that the existing driveway on 98th St. is 175’ south of Central Ave.

- The existing auxiliary right-turn lane length is deficient in length.  The COA DPM section 7-4(I)(7)(iii)(d) Right-turn Lane Design defines the minimum turn lane lengths in Table 7.4.68 Right-turn Lane design Criteria.  A right-turn lane at this location should be at least 350 feet long with 600’-300’ reverse Curve transition.  The exiting turn lane length is approximately 240’ with no transition.

- The proposed access on 98th St. is located within the existing northbound right-turn lane approximately centered in the auxiliary turn lane.  Driveway access within a turn lane is not a normally acceptable condition.  The TIS indicates in the Crash Data section of the report that the highest contributing factor for crashes at this intersection is due to Driver Inattention.  Locating an access point within a turn lane under these conditions is unacceptable due to the location of the access mid-way of a turn lane and apparent poor safety history.

Items #1, minimum access spacing distance from an intersection, and Item #2, minimum length of a turn-lane, will require review and approval of a waiver by the City’s Development Facilitation Team (DFT) before the TIS can be further reviewed for approval.

Item #3, access/driveway within an auxiliary right-turn lane is not covered in the DPM regulations but would be considered a Best Practice and sound Engineering judgement to prohibit a driveway in this location.  Written approval from the City Engineer is needed to document acceptance this atypical driveway treatment.

Please address comment #1 and #2 through DFT waiver request and #3 by approval from the City Engineer. Further review of this TIS will not be performed until acceptable location of the 98th St access is proposed, the lengthening of the north to east right-turn lane is increased to meet the DPM minimums, and the access on 98th St. is not located within an auxiliary right-turn lane.

If you have any questions, feel free to contact me at (505) 924-3362.

Sincerely,

Curtis Cherne, P.E.

City of Albuquerque

Senior Engineer, Planning Dept.

Development Review Services