|  | Date: 09/13/19 |  |  |  |
| --- | --- | --- | --- | --- |
| Project: | Alliance Residential Builders, LLC. for Broadstone Highlands. |  |  |  |
| # Corrective Action Log |  |  |  |  |
| Number | Corrective Action Needed | Date Action Noted | Date Corrected | Date Correction Noted By Inspector |
| 01 | BMP's in place prior to earth disturbing activities. No site corrective actions noted at the time of inspection. No active work at the time of inspection. | 05/10/19 |  | 05/10/19 |
| 02 | Storm event on 05/09-10/19 recorded at: 0.27". Inspection completed on 05/10/19. | 05/10/19 |  | 05/10/19 |
| 03 | Storm event on 05/11/19 recorded at: 0.09". | 05/13/19 |  | 05/13/19 |
| 04 | Install silt fence in the South West corner per Erosion control plan, aprox. 100'. Recent grading has covered or removed the fence in this area. | 05/24/19 |  |  |
| 05 | Sweep sidewalk on the West end just South of the old construction entrance. Install wattles at the base of the silt fence in this area where fabric is torn/ weathered. Aprox. 50'. | 05/24/19 |  |  |
| 06 | Install silt fence to the South, between construction entrances, per Erosion control plan; aprox. 50'. | 05/24/19 | Wattles in place. |  |
| 07 | Stabilize the exit created on the South East side, maintain the cut back behind the curb and sweep sidewalk/ street when complete. | 05/24/19 | Sidewalk swept. | 08/02/19 |
| 08 | City to extend grading limit boundary. Current grading, southwest, has overrun BMP's in place. Replace BMP's per Erosion Control Plan at earliest opportunity. | 06/07/19 |  |  |
| 09 | Provide SWPPP book on site for inspector and city review of project. | 06/07/19 |  | 07/05/19 |
| 10 | Add fractured rock to construction exit, east side. | 06/21/19 | Exit closed. | 07/19/19 |
| 11 | No additional site BMP corrective actions noted at the time of inspection. No active work at the time of inspection, afternoon on Friday the 28th. | 06/28/19 |  | 06/28/19 |
| 12 | No additional site BMP corrective actions noted at the time of inspection. No active work at the time of inspection. Storm event on 06/27/19= 0.05" | 07/05/19 |  | 07/05/19 |
| 13 | Storm events per Weather Underground, "Summit Park": 07/01/19=0.13"; 07/06/19= 0.20" | 07/11/19 |  | 07/11/19 |
| 14 | No additional site BMP corrective actions noted at the time of inspection. Block wall work in progress, south end. | 07/11/19 |  | 07/11/19 |
| 15 | No additional site BMP corrective actions noted at the time of inspection. Corrective action items remain surrounding property boundary: Silt fence repairs/ install, inlet sediment at BMP's on Copper, sediment escape to sidewalks. Inspector unable to locate SWPPP book on site. Storm event on 07/16/19= 0.10" | 07/19/19 |  | 07/19/19 |
| 16 | Add fractured rock to construction exit, east side. Slight trackout on Copper is cumulative of Jaynes and Alliance. Storm event on 07/25/19= 1.23" | 07/26/19 |  | 08/02/19 |
| 17 | Sweep the street: East entrance to West end Inlet on Copper. Storm event: 07/29/19= 0.15" per Weather Underground and nearby gauge. | 08/02/19 |  | 08/05/19 |
| 18 | No additional site BMP corrective actions noted at the time of inspection. Storm event on 08/03/19= 0.10" . East construction exit closed off, west end exit covered by fractured rock. | 08/09/19 |  | 08/09/19 |
| 19 | No additional site BMP corrective actions noted at the time of inspection, despite increased activity: Builders Source, Water Authority, NOEL, Bohannan Huston on site. East end wall in progress, cut back in place. Active trash cleanup during inspection. Storm events: 08/06/19= 0.20"; 08/11/19= 0.08"; per Weather Underground and onsite gauge. | 08/16/19 |  | 08/16/19 |
| 20 | No additional site BMP corrective actions noted at the time of inspection. Storm event: 08/21/19= 0.15" per Weather Underground and onsite gauge. | 08/23/19 |  | 08/23/19 |
| 21 | No additional site BMP corrective actions noted at the time of inspection. | 08/30/19 |  | 08/30/19 |
| 22 | No additional site BMP corrective actions noted at the time of inspection. Storm event: 08/27/19= 0.07"; 09/03/19= 0.05" per Weather Underground and onsite gauge. | 09/06/19 |  | 09/06/19 |
| 23 | No additional site BMP corrective actions noted at the time of inspection. Block wall work in progress, south and west sides near property boundary. Storm events: 09/06/19=0.05"; 09/10/19=0.08" | 09/13/19 |  | 09/13/19 |
| SWPPP Management Signature: Date: |  |  |  |  |