Vancel S. Fossinger, P.E.

Wilson & Company, Inc.

4900 Lang Ave. NE

Albuquerque, NM 87109

Re:	Lincoln Complex, Drainage Master Plan, Engineer’s Stamp

Dated 10-11-2011 (K15/D47)

Dear Mr. Fossinger,

Based upon the information provided in your submittal received 11-13-11, the above referenced report cannot be approved until the following comments are addressed:

- Please use the precipitation data from Zone 2, Table A.2 for the runoff calculations.

- It appears that Pond 3 will encroach into the proposed building.

- There are existing buildings where Pond 1 and 2 are proposed.  Are these buildings being removed?

- Show existing easements on the plan.

- Include on Tables 1 and 3 that the flows are based on the 100-yr/6-hr storm and volumes are based on the 100-yr/24-hr storm.

- Double check the numbers from the Tables against the AHYMO results.

- AHYMO input file shows that the site is within Zone 1, and it should be Zone 2.

- Approval from AMAFCA will be required for the proposed changes.

- New Mexico Department of Transportation approval will be required for any changes to the existing discharges to the NMDOT right-of-way as well as any construction work within the NMDOT right-of-way.

If you have any questions, you can contact me at 924-3695.

Sincerely,

Shahab Biazar, P.E.

Senior Engineer, Planning Dept.

Development and Building Services

C:	File