City of Albuquerqfue

Planning Department

Development Service / Hydrology Section

Date: __7-18-12__

Conference Recap

GRADING & DRAINAGE PRE-DESIGN

Zone Atlas Page No: _L-12 ________

Drainage File: __L12 / D011 _______

Zoning: ________  O-1  ___________

DRB:

Subject:  Cornucopia Center / Greenhouse addition

Street Address (If Known):    2002 Bridge Blvd.  SW (SW Corner of Bridge & Pear)

Subdivision Name: ___________________________________________

Approval Requested:  GP for Building Permit

Attendance: __Greg Olson (COA), John MacKenzie (DMG) ,  Michele Mullen (AIA)

Findings:

- Determine existing runoff conditions and historical discharge points & ponding locations.  The topo survey provided seems to show existing sheet flow leaving the site onto private property to the south and west.  Michele indicated that the flat, site grading historically ponds on-site and on the dirt shoulder along the east side of Pear Street.

- Attempt to limit site discharge rates to 2.75 cfs per acre.

- Also, the new MS4 Permit will require retention of the first 0.47’ of rainfall on site.

- We discussed possible “ponding” locations and minimal grading within the site and around the perimeter of the greenhouse pad.  (provide pad elevation for the Greenhouse)

- Due to its location adjacent to Bridge Blvd., we do not have Storm Drain System records for the area.      (Please provide the layout of adjacent public facilities if site runoff will discharge to Bridge Blvd.)

- Follow up with Transportation section to confirm any site paving requirements.