August 21, 2012

Reza Afaghpour, P.E.

SBS Construction & Engineering, LLC.

P.O. Box 10264

Albuquerque, NM 87184

Re:	Fourplex for Ahmet Tiryaki, Grading & Drainage Plan          (L-18/D078)

509 Valencia Dr. SE			Engineer’s Stamp dated 08-01-12

Dear Mr. Afaghpour,

Based upon the information provided in your submittal received 08-01-12, the above referenced plan cannot be approved for Building Permit until the following comments are addressed:

- The Drainage Management Plan states that the ponds will detain “+265 cf” of flow. Provide pond calculations demonstrating how this is accomplished.

- The DMP also indicates the existing condition to be Treatment B, however vacant lots are defined in DPM Chapter 22 as Treatment C.  Please recalculate flows with Treatment C as the existing condition.

- Show on the plan where Roof Flows will discharge and how they will be routed.

- Show/label all ponding areas and flow arrows on the plan.

- The water-block along Valencia Drive appears to be inadequate.  Provide sidewalk, flowline, and curb elevations on the plan along Valencia Drive.

- Provide additional spot elevations along the southern sidewalk since curb will be flush.

- Include additional spot elevations along the northern boundary line to the east of the proposed retaining wall/fence, and provide spot elevations in the landscape areas.  Grades provided appear to be over 1’ higher than existing grades at the property line.

- The plan shows both a retaining wall and a 6 foot high fence in the same location on north & west boundary.  Please clarify this in legend or in a detail drawing on the plan.

- The south edge of the plan shows a retaining wall starting ~12’ from the SW corner, however the plan shows the same wall symbol going east for >100’;  clarify your intent.

- The last sentence under the “Proposed Conditions and On-Site Drainage Management Plan” heading is illegible and unclear.  Charleston Street is not within the vicinity of this drainage plan.

- Clarify grading at the 8” PVC pipe crossings near the southern sidewalk.

- Correct several spelling errors on the plan notes (eg: “CURP OPENNING” & “exiting”).

Albuquerque’s MS4 Permit became effective March 1st, 2012.  All Grading and Drainage Plans and Drainage Reports will have to comply with the requirements of the new permit (http://www.cabq.gov/planning/landcoord/documents/EPA-NMS000101-FinalPermit.pdf).

If you have any questions, you can contact me at 924-3994.

Sincerely,

Gregory R. Olson, P.E.

Senior Engineer

Orig:	Drainage file  E12/D078

c.pdf	Addressee via Email  AECllc@aol.com