March 13, 2019

Matthew Vallejos, CPESC – In Training

Green Globe Environmental,  LLC

P.O. Box 400

Los Lunas, NM 87031

Re:	Juan Tabo Hills Estates (Volterra)- DR Horton Homes

Erosion and Sediment Control Plan

Engineer’s Stamp Date –no stamp- (M21E018_H)

Dear Mr. Vallejos,

Based upon the information provided in your submittal received 3-11-19, the above referenced plan cannot be approved to be included in the SWPPP or for NOI documentation review for grading and Building Permit until the following comments are addressed:

1.  The plan is not sealed by a Registered Engineer in New Mexico or a Certified Professional in Erosion and Sediment Control.  I reviewed the “In-training Certification” on Envirocert’s website and a CPESC- In Training does not meet the standards in the Drainage Ordinance.

2.  A filter sock, wattles or J-Hook silt fence at regular intervals is required in the cutback area on streets with a longitudinal slope steeper than 2.5% to help prevent erosion.  Streets identified meeting this condition are: Fred Harvey-3.3%, Manzano Vista 2.66% and 3.2% and Sandia Sunset 2.6%.  Slopes were obtained from the Work Order construction plans.

3.  The cutback is too small and the wrong shape in the Cut-back curb detail.  The cutback should resemble a V-ditch with the low-point away from the curb.  A detail developed with home builders accompanies this letter.

If you have any questions, you can contact me at 924-3420.

Sincerely,

Curtis Cherne, P.E.

Principal Engineer, Stormwater Quality

Planning Dept.