SWPPP - The SWPPP still wasn’t “up-to-date” in violation of CGP 7.1. Several items from the 2/2/26 inspection had been addressed, but new deficiencies were identified during the 3/11/26 inspection.

- NOI update – The updated NOI, adding Rhonda Reynolds as the operator point of contact on 3/6/26, wasn’t in the SWPPP.

- SWPPP Modifications - Records showing the dates of SWPPP modifications, including the name of the person authorizing the changes and a brief summary of the changes, were missing in violation of CGP 7.4.2. The SWPPP Maps were also difficult to follow. The Phase 2 ESC Plan, with the engineer’s stamp dated 9/6/22, was included in the SWPPP during the 3/11/26 inspection, but it was unclear which NOI or SWPPP change it pertained to, or who authorized it.

- The SWPPP Map (aka ESC Plan) had been reduced and wasn’t legible. The originally designed location of the silt fence wasn’t discernible. The person authorizing each change was missing.

- Missing reports - The Corrective action report dated 2/27/26 was missing.

Required Mitigation: The NOI certified on 3/6/26 must be added to the SWPPP along with records showing the dates of all SWPPP modifications, including the name of the person authorizing the changes and a brief summary of the changes as required by CGP 7.4.2. The records must include all of phases 2 and 3, but not Phase 1, NMR10033A & B, which began on 06/24/2020, since it was terminated on 02/18/22, more than three years ago per CGP 4.7.4. The records should include a map showing the area of permit coverage each time an NOI was filed or corrected beginning with Phase 2, NMR1004XE, originally certified on 7/8/22, and terminated by Laura Gutierrez on 6/16/2025. The records should also include Phase 3 NMR10069Y, which began on 3/25/24 as a 2-acre improvement to the pond west of Phase 2, and on 3/20/25, it was expanded to 15 acres, including all of Phases 2 and 3.

The maps and records should be collated, perhaps adding a key map showing the past phases, areas of NOI coverage, dates, and identifying areas where construction and stabilization are complete. Also include a legible map of current conditions and the history of BMP installation, maintenance, and resets, including the date and the person who authorized the changes per CGP 7.4.3. Include the recent corrective action report in the SWPPP.

records showing The SWPPP, including the SWPPP Map, Self-Inspection Reports, and Corrective Action Reports, must be kept up-to-date per CGP 7.1 and available on-site per CGP 4.7.3, 5.4.3, and 7.3. If an on-site location is unavailable to keep the SWPPP when no personnel are present, notice of the plan’s location must be posted near the main entrance of your construction site, CGP 7.3.